Firmware update procedures

- Connect the 2-Port PM, AD2HPMREU, to a PM compatible SATA port

- Unzip the file 109xFWUpgv10.zip and save all the files to a folder

- Open the folder where the files are located and double click on the file 109xFWUpg.exe to launch the utility

- Select ASMT109x-Config in the Device list pull down menu

- Click on this button  and select the BIN file 141027_00_18_08.bin

- Click Update Firmware button

- When shown “Please wait for a while”, click OK button

- When shown “Success”, the update is completed.