慧榮科技股份有限公司

研發設計/變更版本管制表單				單版次: A

| 產品名稱: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 用途: 修改功能 |  |  |  |  |  |  |  |  |  |
| 隸屬專案: Zoom series |  |  |  |  |  |  |  |  |  |
| 此版本為: □新發行 ■設計變更 |  |  |  |  | 日期: 02/23/2010 |  |  |  |  |
| 資料類別歸屬: □IC H/W ■IC F/W □系統H/W □系統 F/W ■應用程式 □其他 |  |  |  |  |  |  |  |  |  |
| 版次: | #### H/W版次 |  | F/W版次 |  |  |  | S/W版次 |  | 其他 |
|  |  |  | ISP2242AB_J0222.BIN |  |  |  | J0208A |  |  |
| 內容概述: SW: FW: - Fix get status command issue on Hynix 32nm. - Check minimum spare block after adjusting capacity when initial card. - Modify controller to flash driving strength to be 3 level bases on flash number. |  |  |  |  |  |  |  |  |  |
| 建議通知客戶群組 |  |  |  |  |  |  |  |  |  |
| 需知會相關人員 |  | Mars, Jacky , Janice, Vincent, Johnson , Coco, jack |  |  |  |  |  |  |  |
| 部門主管 |  | 單位主管 |  | 設計/變更 |  | 功能驗證 |  | 經辦人 |  |

接前頁內容概述：

| Program 變更事項 | Controller | 檔名 |
| --- | --- | --- |
| I0410 |  |  |
| New Firmware ISP | SM2242AB | ISP2242AB_I0415.BIN |
| I0504 |  |  |
| New Firmware ISP MPTool | SM2242AB | ISP2242AB_I0429.BIN sm224MPTool.exe |
| I0504 |  |  |
| New Firmware ISP MPTool | SM2242AB | ISP2242AB_I0505BIN sm224MPTool.exe |
| I0505 |  |  |
| New Firmware ISP MPTool | SM2242AB | ISP2242AB_I0505ABIN sm224MPTool.exe |
| I0505 |  |  |
| New Firmware ISP MPTool | SM2242AB | ISP2242AB_I0515BIN sm224MPTool.exe |
| I0601 |  |  |
| New Firmware ISP MPTool | SM2242AB | ISP2242AB_I0527D.BIN sm224MPTool.exe |
| I0604 |  |  |
| New Firmware ISP MPTool | SM2242AB | ISP2242AB_ I0604.BIN sm224MPTool.exe |
| I0611 |  |  |
| New Firmware ISP MPTool | SM2242AB | ISP2242AB_I0611B.BIN sm224MPTool.exe |
| I0617 |  |  |
| New Firmware ISP MPTool | SM2242AB | ISP2242AB_ I0615B.BIN sm224MPTool.exe |
| I0630 |  |  |
| New Firmware ISP MPTool | SM2242AB | ISP2242AB_ I0629.BIN sm224MPTool.exe |
| I0703 |  |  |
| New Firmware ISP MPTool | SM2242AB | ISP2242AB_I0703B.BIN sm224MPTool.exe |
| I0713 |  |  |
| New Firmware ISP MPTool | SM2242AB | ISP2242AB_I0713.BIN sm224MPTool.exe |
| I0722 |  |  |
| New Firmware ISP | SM2242AB | ISP2242AB_I0722.BIN sm224MPTool.exe |

| I0728 |  |  |
| --- | --- | --- |
| New Firmware ISP MPTool | SM2242AB | ISP2242AB_I0722.BIN sm224MPTool.exe |
| I0803 |  |  |
| New Firmware ISP MPTool | SM2242AB | ISP2242AB_I0803B.BIN sm224MPTool.exe |
| I0812 |  |  |
| New Firmware ISP MPTool | SM2242AB | ISP2242AB_I0811.BIN sm224MPTool.exe |
| I0818 |  |  |
| New Firmware ISP | SM2242AB | ISP2242AB_I0811.BIN |
| I0831 |  |  |
| New Firmware ISP MPTool | SM2242AB | ISP2242AB_I0811.BIN sm224MPTool.exe |
| I0908 |  |  |
| Flash Database | SM2242AB | Flash.set |
| I0914 |  |  |
| New Firmware ISP MPTool | SM2242AB | ISP2242AB_ I0914.BIN sm224MPTool.exe |
| I0918 |  |  |
| New Firmware ISP MPTool | SM2242AB | ISP2242AB_ I0916.BIN sm224MPTool.exe |
| I1002 |  |  |
| New Firmware ISP MPTool | SM2242AB | ISP2242AB_I1002.BIN sm224MPTool.exe |
| I1009 |  |  |
| New Firmware ISP MPTool | SM2242AB | ISP2242AB_I1009.BIN sm224MPTool.exe |
| I1102 |  |  |
| New Firmware ISP MPTool | SM2242AB | ISP2242AB_I1030.BIN sm224MPTool.exe |
| I1106 |  |  |
| New Firmware ISP MPTool | SM2242AB | ISP2242AB_I1106.BIN sm224MPTool.exe |
| I1127 |  |  |
| MPTool | SM2242AB | sm224MPTool.exe |
| I1202 |  |  |
| MPTool | SM2242AB | sm224MPTool.exe |
| I1203 |  |  |
| MPTool New Firmware ISP | SM2242AB | sm224MPTool.exe ISP2242AB_I1203.BIN |
| I1222 |  |  |
| MPTool New Firmware ISP | SM2242AB | sm224MPTool.exe ISP2242AB_I1222.BIN |
| J0105A |  |  |
| New Firmware ISP | SM2242AB | ISP2242AB_J0105.BIN |
| J0208A |  |  |
| New Firmware ISP | SM2242AB | ISP2242AB_J0208.BIN |

| FW 變更事項 | Controller | 檔名 |
| --- | --- | --- |
| I0415 |  |  |
| 1. Release the first version of SM2242 | SM2242AB |  |
| I0504 |  |  |
| - Add Cache Input Flash feature. - Fix Power Cycling issues. | SM2242AB |  |
| I0505 |  |  |
| 1. Add sequential read feature. - Fix Write Protect bug. | SM2242AB |  |
| I0505A |  |  |
| - Fix the memory locate issue. - Modify the firmware version of EC command is from Write Protect page. | SM2242AB |  |
| I0515 |  |  |
| - Fix burn-in failure in MicronL63A as cache enable. - Fix time out issue in dos mode, make SATA transport protocol to be UDMA mode not PIO mode. | SM2242AB |  |
| I0601 |  |  |
| - Fix device detect failure in RocketRAID 2310-Marvell 88SX7042-BDU1 and RocketRAID 3510-Marvell 88SX6941-BCZ1. - Add new function: MPTool would load fitting paired page table for different flash. - Modify read flow for Ecc handle. - Modify write flow for performance. | SM2242AB |  |
| I0604 |  |  |
| - Fix BIOS detection fail in PC24 (NVIDIA nForce 6309), PC34 (VIA VT8251) - Fix Speed is slow when install system by ghost on Atom 3 (Intel 82801GB(ICH7/R)). - Fix HY27UV08BG5A download ISP fail. - Fix miss strong page table for 256 page flash. - Fix MP fail by PATA interface on Non-SMI Tester. | SM2242AB |  |
| I0611 |  |  |
| - Support Micron 29F128G - Support SM2242 SSD Life Guard. | SM2242AB |  |
| I0617 |  |  |
| - Modify flash data and command driving to be 12mA for 2 CE and 16mA for greater than or equal to 4 CE - Fix pretest can’t reference run time bad. | SM2242AB |  |
| I0629 |  |  |
| - Add new CID Table option: Wait another CE ready before send read command in interleave mode by bit 1 of offset 0x0A in CID table and base on SpecialDB.SET. - Modify SATA interrupt flow and EC command for SATA compatibility. | SM2242AB |  |
| I0703 |  |  |
| - Add Flash IO driving setting function base on 0x28 (normal mode or SLC of hybrid mode or hybrid MLC mode) and 0x29 (MLC of hybrid mode) of CID table. For Micron L63A, L63B, L62A. - Force enable cache write. - Update date base to fix Strong-weak page table error selection for JS29Fxxx08xAMDA(INTEL L63B). | SM2242AB |  |
| I0713 |  |  |
| - Fix total FBlock larger than 4k may lost child block after building link table. - Add new function to check ISP block ECC status. If there are some error, it will force to run ROM code. | SM2242AB |  |
| I0722 |  |  |
| - Fix program and erase command error in 2 plane mode for IM 4k Flash. - Fix block “0” may be use after backup system block when power on. - Fix reference run time bad of pretest option would fail as new bad block is different address block. - Fix the bug that SECURITY ERASE UNIT command will be accepted when User password is valid | SM2242AB |  |
| I0728 |  |  |
| - Fix read and erase command wait all flash busy in 2 plane mode for IM L63B, L62A Flash. - Fix read data of smart command (0xD1) fail on HDAT2. - Add BIST L mode. | SM2242AB |  |
| I0803 |  |  |
| - Fix SLC + MLC (K9K8G + K9GAG) would fail in initial card. - Fix K9GBG would result abnormal bad block in burn-in test. - Add security command: EraseUnit. |  |  |
| I0811 |  |  |
| - Add Scan2242H.bin to scan original bad block before initial card. - Add RemarkOrig2242H.bin to recover original bad block. - Add function: Block Early Retire. |  |  |
| I0818 |  |  |
| - Add device capacity adjustment. - Add 48 bit feature set. |  |  |
| I0831 |  |  |
| - Support HPA feature set. - Add HW write protect on P40, and Quick Erase on P41. Enable this feature by MP Tool. |  |  |
| I0914 |  |  |
| - Support ONFI IM Flash without ONFI table (43nm SLC and MLC). - Modify SMART command Read Data (0xD0) to add Total Bad count, New Bad count and CRC error counter. - Fix some info can’t be update in SSD Life Guard. - Extend average erase count for SLC Flash. - This firmware version should match with later SSDLifeGuard I0911 and WLTool I0911. |  |  |
| I0916 |  |  |
| - Modify SMART command Read Data (0xD0): Only record CRC error count during one power cycle. |  |  |
| I1002 |  |  |
| - Fix copy compare error on burn-in test because some issues are due to fix write performance is unstable on HD Tune on I0925. |  |  |
| I1009 |  |  |
| Fix the ATA command: - Abort CFA ERASE SECTORS. - Fix CFA TRANSLATE SECTOR command. - Avoid crash and abort command for PACKET command. - Avoid crash and abort command for SERVICE command. |  |  |
| I1030 |  |  |
| - Support internal interleave for SAMSUNG flash. - Disable LBA48 when device size small than 8G Bytes avoid host compatibility issue. - Report support TRIM command and non-rotate device. - Report not support read write log. - Modify LED Pin define. P40:LED for H/W WP, P42: LED for SATA PHY. - Support customer adjust spare block threshold for SMART command return a bad/fail status by Spare Threshold on MPTool. |  |  |
| I1106 |  |  |
| - Support runtime detect HW write protect. - Hold at least 1s to active HW quick erase. - Modify report spare block from run time spare block to total spare block (include child block) in READ DATA (0xD0) of SMART command. |  |  |
| I1203 |  |  |
| - Support Toshiba-43nm SLC, TC58NVG0S3ET, TC58NVG1S3ET, TC58NVG2S3ET. - Support Micron-M61A, 29F16G08AFABA. - Support Intel L63B TSOP Flash with two different Flash ID. - Add error handle as out of spare block. |  |  |
| I1222 |  |  |
| - Support Hynix H27UDGV5ATR, H27UCGV5ATR. - When read ECC fail, issue one plane read command to retry for 2 plane read flash. - Support Toshiba TC58DVG3S0ETA00, TH58DVG5S0ETA20 and TH58DVG4S0ETA20. - When system power lower than 2.3V, skip all program, and make device to be read only. |  |  |
| J0105 |  |  |
| - Fix lost ID table after wearleveling. |  |  |
| J0208 |  |  |
| 1. Support Hynix H27UDGV5ATR, H27UCGV5ATR. 2. When read ECC fail, issue one plane read command to retry for 2 plane read flash. 3. Support Toshiba TC58DVG3S0ETA00, TH58DVG5S0ETA20 and TH58DVG4S0ETA20. 4. Fix Samsung K9K8G, K9KAG read data issue in internal interleave mode. - Enable link table only use strong page in MLC Flash. - Add ISP Block Error Handle (Load ISP code with ECC failure, assign another ISP block to current ISP Block). - Disable Hybrid mode. |  |  |

| SW 變更事項 | Controller | 檔名 |
| --- | --- | --- |
| I0505 |  |  |
| 1. Integrated SM22X MPTool first version | SM2242AB |  |
| I0515 |  |  |
| 1. Modify PreScan for SM2242 | SM2242AB |  |
| I0601 |  |  |
| 1. Add Pretest StrongWeak Table Binary. | SM2242AB |  |
| I0604 |  |  |
| 1. Add Production Log Option. | SM2242AB |  |
| I0611 |  |  |
| 1. Modify wear-leveling to 64 | SM2242AB |  |
| I0617 |  |  |
| 1. Manually modify database | SM2242AB |  |
| I0630 |  |  |
| 1. Add Special Database \\ FlashDB\2242AA\ SpecialDB.SET 2. Add CID Setting “SM2242ByteA” in CID[0x0A] 3.Modify Model Name & Serial Number padding 0x0 -> 0x20 | SM2242AB |  |
| I0703 |  |  |
| 1.Remove Special Database \\ FlashDB\2242AA\SpecialDB.SET 2.Remove CID Setting “SM2242ByteA” in CID[0x0A] 3.Add Hybrid Option to CID[0x28] CID[0x29]. |  |  |
| I0728 |  |  |
| Dummy revision |  |  |
| I0803 |  |  |
| Lower the clock to 60 MHz when CE per channel >4. |  |  |
| I0812 |  |  |
| Remove Clock limit on CE per channel > 4. Change PATA Driving into CID[0x23]. |  |  |
| I0818 |  |  |
| Dummy revision |  |  |
| I0831 |  |  |
| Add QC Procedure. Add save bad binary into production. |  |  |
| I0908 |  |  |
| Add flash below in databse 29F32G08CEABA 29F16G08AAMD2 H27UBG8U5ATR H27UCG8V5ATR |  |  |
| I0914 |  |  |
| Add Hybrid Flash Select Function. |  |  |
| I1002 |  |  |
| Change MP Tool resolution. |  |  |
| I1102 |  |  |
| Add Spare Threshold Option. |  |  |
| I1106 |  |  |
| Modify SM2242 Spare check. |  |  |
| I1127 |  |  |
| Enable PreTest Interleave mode. |  |  |
| I1202 |  |  |
| Fix Hybrid mode check spare fail. |  |  |
| I1203 |  |  |
| Add Disable PATA Terminator. |  |  |
| I1222 |  |  |
| Add Fix Toshiba ID shift by Database. |  |  |

| ISP Checksum |  |
| --- | --- |
| ISP-2242AB-I0415.BIN | 006712A8 |
| ISP2242AB_I0505.BIN | 0068F3A9 |
| ISP2242AB_I0505A.BIN | 0068C1C2 |
| ISP2242AB_I0515.BIN | 006BE8D2 |
| ISP2242AB_I0527D.BIN | 006D2E1D |
| ISP2242AB_I0604.BIN | 006D4A64 |
| ISP2242AB_I0611B.BIN | 006DBACA |
| ISP2242AB_I0615B.BIN | 006E4041 |
| ISP2242AB_I0625.BIN | 006EF63C |
| ISP2242AB_I0629.BIN | 006F143B |
| ISP2242AB_I0703B.BIN | 006F989E |
| ISP2242AB_I0713.BIN | 006FAE9F |
| ISP2242AB_I0722B.BIN | 006FBF84 |
| ISP2242AB_I0727C.BIN | 006CCC87 |
| ISP2242AB_I0803B.BIN | 006EAC05 |
| ISP2242AB_I0811.BIN | 006F15F7 |
| ISP2242AB_I0818.BIN | 0070AF7A |
| ISP2242AB_I0831.BIN | 0070A6C0 |
| ISP2242AB_I0914.BIN | 00709BEC |
| ISP2242AB_I0916.BIN | 006FE284 |
| ISP2242AB_I1002.BIN | 00703B47 |
| ISP2242AB_I1030.BIN | 006FDBC7 |
| ISP2242AB_I1106.BIN | 006FF992 |
| ISP2242AB_I1203.BIN | 00706AA0 |
| ISP2242AB_I1222.BIN | 00742319 |
| ISP2242AB_J0105.BIN | 00744B92 |
| ISP2242AB_J0208.BIN | 00725057 |