慧榮科技股份有限公司

研發設計/變更版本管制表單				單版次: A

| 產品名稱: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 用途: 修改功能 |  |  |  |  |  |  |  |  |  |
| 隸屬專案: Zoom series |  |  |  |  |  |  |  |  |  |
| 此版本為: □新發行 ■設計變更 |  |  |  |  | 日期: 10/24/2013 |  |  |  |  |
| 資料類別歸屬: □IC H/W ■IC F/W □系統H/W □系統 F/W ■應用程式 □其他 |  |  |  |  |  |  |  |  |  |
| 版次: | #### H/W版次 |  | F/W版次 |  |  |  | S/W版次 |  | Check Sum |
|  |  |  | ISP2246 M1024A |  |  |  | M1024A |  | 0x00EFFEAD |
| 內容概述: F/W: - Fix flush cache & HDMA over assignment bug in write flow. - Support SCT write same function. - Keep the empty blocks which is the source of the active block non-complete word-line pages to avoid data lose when power-cycling. - Fix Lite-on script issue. - Support “31” command for Netcom. - Exit background clean if any host command in except swapping ecc fail block to avoid command timeout. - Shut down flash core power in device sleep mode. - Support scale-down external-interleave way. S/W: 1. Add 5 CID option "TranscendTSBTek", "AHCCMDTCompatible", "DCO","SCT","PHYEvent" 2. Add CID option, "Erase Count threshold" and "Spr Bk number threshold" for "TSB Tek" 3. Modify SM2246 "DumpOrigBad" and "RDT data" 4. Modify "Update Tester" function 5. Add function for Aptos RDT initail flow (FT1/FT2/FT3/EQA) 6. Add QC mode for Kingmax. 7. Update Aptos MP flow for SM2246 8. Add QC suberror code display on error code define button. 9. add QC item check for Netcom request |  |  |  |  |  |  |  |  |  |
| 建議通知客戶群組 |  |  |  |  |  |  |  |  |  |
| 需知會相關人員 |  | Vincent, Aaron, Roger, Bassa |  |  |  |  |  |  |  |
| 部門主管 |  | 單位主管 |  | 設計/變更 |  | 功能驗證 |  | 經辦人 |  |

接前頁內容概述：

| Program 變更事項 | Controller | 檔名 |
| --- | --- | --- |
| M0524 |  |  |
| MPTool New Firmware ISP |  | sm22XMPToolM0520A.exe ISP2246.BIN |
| M0531 |  |  |
| MPTool New Firmware ISP |  | sm22XMPToolM0528A.exe ISP2246.BIN |
| M0607 |  |  |
| MPTool New Firmware ISP |  | sm22XMPToolM0607A.exe ISP2246.BIN |
| M0610 |  |  |
| MPTool New Firmware ISP |  | sm22XMPToolM0610A.exe ISP2246.BIN |
| M0618 |  |  |
| MPTool |  | sm22XMPToolM0610A.exe |
| M0621 |  |  |
| MPTool New Firmware ISP |  | sm22XMPToolM0618A.exe ISP2246.BIN |
| M0626 |  |  |
| MPTool New Firmware ISP |  | sm22XMPToolM0626A.exe ISP2246.BIN |
| M0628 |  |  |
| New Firmware ISP |  | ISP2246.BIN |
| M0703 |  |  |
| New Firmware ISP |  | ISP2246.BIN |
| M0709 |  |  |
| New Firmware ISP |  | ISP2246.BIN |
| M0726 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM0723A.exe |
| M0808 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM0808A.exe |
| M0809 |  |  |
| MPTool |  | sm22XMPToolM0808B.exe |
| M0823 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM0815A.exe |
| M0830 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM0829A.exe |
| M0906 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM0906A.exe |
| M0918 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM0918A.exe |
| M0926 |  |  |
| New Firmware ISP |  | ISP2246.BIN |
| M1003 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM1001A.exe |
| M1007 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM1003A.exe |
| M1011 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM1009B.exe |
| M1024 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM1024A.exe |

| FW 變更事項 | Controller | 檔名 |
| --- | --- | --- |
| M0524A |  | ISP2246.BIN |
| 1. Fix refresh time-out issue 2. Support Mobile Mark power consumption measurement 3. Fix BIOS detect fail of NB04(FUJITSU AH550) 4. Set SMART thermal report under 69 degree |  |  |
| M0531A |  | ISP2246.BIN |
| 1) Support Mobile Mark power consumption measurement 2) Compatibility issue a) NB42, NB57 and NB 53 are host’s compatibility issue, it can pass by disable “FW partial to slumber”. 3) Fix power-cycling issue found from Netcom 4) Fix ECC early move bug 5) Change DRAM setting 6) Change retry index definition, up to 20 |  |  |
| M0607A |  | ISP2246.BIN |
| 1. Support 4ch 8way flash config. 2. Fix a power-cycling Dram backup bug. 3. Remove an unreasonable debug code in read-retry. 4. Update flash retry ID. |  |  |
| M0610A |  | ISP2246.BIN |
| 1. Update dram power-mode resume setting.(Dram fail issue) 2. Adjust the SPOR backup criterion. (SMI SPOR issue) |  |  |
| M0621A |  | ISP2246.BIN |
| 1. Enable dram compensation 2. Add vendor smart cmd. 3. Fix background clean bug. |  |  |
| M0626A |  | ISP2246.BIN |
| 1. Change power mode DramSetting 2. Fix Smart Info bug. 3. Fix NetCom powercyling fail issue. 4. Fix SMI SPOR fail issue. |  |  |
| M0628A |  | ISP2246.BIN |
| 1. Enhance command latency. 2. Enhance IO-meter IOPS. 3. Enhance CrystalDisk 512k read. 4. Fix a potential risk in standby mode. (VCT S4 BlueScreen issue- FWM0626A) 5. Fix a 4plane read-retry bug. 6. Fix a relink bug(Netcome power-cycling issue). |  |  |
| M0703A |  | ISP2246.BIN |
| 1. Fix bugs found in M0628A. 2. Reduce cached dram data from 16MB to 2MB. 3. Update SMART command ID. 4. Fix trim bug. 5. Fix Sata Error handle bug. |  |  |
| M0709A |  | ISP2246.BIN |
| 1. Adjust IPM time out threshold to improve pcmark score. 2. Fix SPOR miss-compare issue. 3. Fix swap bad block bug. 4. Fix trim bug. |  |  |
| M0726A |  | ISP2246.BIN |
| 1. Set flash IO to EDO mode for Toshiba & Sandisk flash read-retry sequence. 2. Modify Sata-Err-handle criteria. 3. Switch to dummy write if pure spare decrease to zero, and do not mark-bad anymore. 4. Disable auto D2H when reset cpu. 5. Modify SMART command. 6. Extend write log page to 32. 7. Fix strong page size bug. 8. Remove flash under driving strength setting iff Samsung & Toshiba flash. 9. Set timer interrupt from 12 second to 10 second. |  |  |
| M0808A |  | ISP2246.BIN |
| 1. Update smart command info. 2. Add JTAG wake up cpu function in IPM and standby mode. 3. Fix pretest bug for manual toggle flash. 4. Reset max correctable error count in ChkBlockRestECCFail() 5. Add gFlashDrive variable in parameter page for adjusting flash driving. 6. Add sata generation and cache program option in CID. 7. Add RDT function. 8. Modify IPM dram setting. |  |  |
| M0815A |  | ISP2246.BIN |
| 1. Increase dram clock from 330Mhz to 400Mhz 2. Add RDT, PreScan, and RestoreBad function. 3. Toshiba A19nm(A2 cmd disable so far) and Intel 20nm support. 4. Strong page only for whole drive support. 5. Further overdrive L85A flash IO. |  |  |
| M0830A |  | ISP2246.BIN |
| 1. Increase dram clock from 330Mhz to 400Mhz 2. Add RDT, PreScan, and RestoreBad function. 3. Toshiba A19nm(A2 cmd disable so far) and Intel 20nm support. 4. Strong page only for whole drive support. 5. Further overdrive L85A flash IO. |  |  |
| M0906A |  | ISP2246.BIN |
| 1.Add Samsung27nm MLC retry table. 2.Modify IM20nm retry policy, if the spare data become 0xFF, then switch to the next Vth. (M0830A L84 SPOR fail issue) 3.Fix build-link bug.(only occur on ch8way) (M0830A TSB19 4ch8way Netcom SPOR issue) - Do background task iff EC command finished. - Wait flash command queue down before resetting channel in sata error handle. - Fix GetBlockSerial function bug.(M0830A SS21nm SPOR issue) |  |  |
| M0918A |  | ISP2246.BIN |
| 1. Disable dram compensation.(KingMax burning issue) 2. Fix a wearleveing bug. (M0906A VCT SD-Stress issue) 3. Fix seek, read verify sector, and RW multiple command bug. 4. Fix a build F2H bug for act1 block which is also a powe-cycling tail block. (M0906A VCT SMI SPOR issue) 5. Backup all the end page of active block for power-cycling.(Netcom abnormal markbad issue) 6. Modify Micron 20nm(L84, L85) read retry set from 4 to 8. 7. Issue flash reset command before retry sequence. 8. Skip program dummy weak page for IM20nm flash. 9. Modify VDT40 power fail policy, While(1) iff VDT40 &VDT27 are equally true. 10. Add Toshiba A19nm SLC-mode block read-retry. 11. Lower schmitt trigger windows (level = 1) for 8CE configuration.( Transcend 8CE 2.5’’ demo board issue) 12. Support Samsung & Hynix dram. 13. Support LTS. 14. Update RDT function. 15. Add a vendor command for flash chunk read. |  |  |
| M0926A |  | ISP2246.BIN |
| High Priority: 1. Reset AHB before clearing dpp in DPS interrupt, and set-clear TSB buffer flag in sata error handler.(VCT M0918A issue item3,19 and Q寶 S4 issue). 2. Don’t reset flash in DisableReadRetry().(Hynix flash don’t accept this action before random data out) 3. Force re-build all the F2H of tail block whose EOB was ecc failed. (M0918A SMI SPOR issue). 4. Fix a power-cycling bug about backup the tail page of active block. 5. Change to new seed table. 6. Erase one empty map block per time in GenNextMapFBlock() to avoid power-cycling issue. 7. Reset the valid page count as 0xFFFF for bad block in relink. (already marked bad but haven’t save map) Low Priority: 8. Speed up SDB flow 9. Fix NCQ Write FUA bug. 10. Add Write Multiple FUA and Write DMA FUA 11. Support write cache enable/disable. 12. Fix VDT check bug. 13. Set bus div1 for 4CE and 8CE. 14. Change g16PushWriteTH for 4x8 config. 15. Issue multi-plane ALE instead of single-plane ALE in read-retry. 16. Add g32GRTPECycle. |  |  |
| M1003A |  | ISP2246.BIN |
| Low Priority: 1. Fix sata-error handle bug. 2. Add Micron20nm SLC read-retry table. 3. Add Dram 380Mhz option. 4. Fix DCO bug. 5. Add RDT loop function. 6. Enable write cache as a default. 7. Fix an A2 read bug for one plane flash. 8. Modify dram VDT from 1.4 to 1.3v. 9. Fix read_verify_sector &Seek command bug. 10. Support DMA setup auto activate. |  |  |
| M1003A |  | ISP2246.BIN |
| Low Priority: 1. Fix RDT bug (only update ISP2246SelfTest.bin) |  |  |
| M1003B |  | ISP2246.BIN |
| 1.Set Schmitt trigger window as 1 for DQS, ALE, CLE, WE, RE, and data pin. 2.Update RDT info. |  |  |
| M1011A |  | ISP2246.BIN |
| 1.Modify flash “Dll Adj” training method. 2.Passing Dell 1.6 script item except “DST Long continues after power-on”. 3.Disable all flash CE if channel is idle. 4.Do not finish partial clean in trim function for passing Win8 WHCK, if hit this situation we need save map before erasing any empty block. 5.Fix a power-cycling bug about backup the tail page of active block. 6.Add Command Support Option(CID 0x4c ) |  |  |
| M1024A |  | ISP2246.BIN |
| 1. Fix flush cache & HDMA over assignment bug in write flow. 2. Support SCT write same function. 3. Keep the empty blocks which is the source of the active block non-complete word-line pages to avoid data lose when power-cycling. 4. Fix Lite-on script issue. 5. Support “31” command for Netcom. 6. Exit background clean if any host command in except swapping ecc fail block to avoid command timeout. 7. Shut down flash core power in device sleep mode. 8. Support scale-down external-interleave way. |  |  |

| SW 變更事項 | Controller | 檔名 |
| --- | --- | --- |
| M0528A |  | sm22XMPToolM0528A.exe |
| 1.Seperate SM2244 and SM2246 Display window individually. 2. Add SM2246 CID option [0x01] bit6 Partial2Slumber. 3. Add flash ID to Pretest header 0x3A~0x3F from flash DB. |  |  |
| M0607A |  | sm22XMPToolM0607A.exe |
| 1. Fix AutoTest for SM2246 2. Disable DRAM file setting for SM2246(Current SM2246 didn't need DRAM setting file,Aaron request) 3. Add FormFactor combo select item for SM2246 Driving setting using. 4. add 4 kinds of Driving setting to CID driving setting for SM2246. 5. Add check procedure on flash CE number for SM2246. 6. Fix Show TagPage flow 7. In SM2246 Flash DB, divide driving setting into four kinds(a.EVB, b. 2.5Inch, c.mSATA , d.NGFF) |  |  |
| M0610A |  | sm22XMPToolM0610A.exe |
| 1. Fixed flow of check CE number within all channels on pretest procedure |  |  |
| M0618A |  | sm22XMPToolM0618A.exe |
| 1. Fix Flash ID judgment method. 2. when do vendor command F1/28 extend timeout. 3. Modify "F_opt2" as 2 digits for new option. 4. Fix "read retry flash id" fail. |  |  |
| M0626A |  | sm22XMPToolM0626A.exe |
| 1. Add flow of check Read Flash ID to fix that vendor command interrupt. |  |  |
| M0723A |  | sm22XMPToolM0723A.exe |
| 1. Fix read Global page function on debug mode 2. Add check Sum function , in setting.set [OPTION] ENABLECOMPCHKSUM=1 3. Fix Pretest button on debug mode, it can download MPISP for SM2246. 4. Update CID setting display 5. Modify Strong page table flow (256=>1024 bytes) |  |  |
| M0808A |  | sm22XMPToolM0808A.exe |
| 1. add show Pretest header in CID option on DialogReadCID. 2. add new flash database item, flash output driving for SM2246 3. add Max SATA speed CID option 4. add DRAM test for SM2246 5. add do prescan on SM2246, in normal initial procedure and in page diagnosis "Bad Block (Original) Diagnosis". 6. show SelfTest page in SM2246 7. add En cache program CID option byte1 bit1 8. compare with FlashID(F0/20) and ID table when verify card size |  |  |
| M0808B |  | sm22XMPToolM0808B.exe |
| 1. modify verify card size,compare totla data sector with FlashID buffer(F0/20) and ID table Word 100~103 |  |  |
| M0815A |  | sm22XMPToolM0815A.exe |
| 1.Add "Other setting" button for extra CID setting page using on parameter page. 2.Add new CID setting, adjust temperature. 3.Update ReadCID page display. 4.Remove not using setting on parameter page. |  |  |
| M0829A |  | sm22XMPToolM0829A.exe |
| 1. Fixed the code about warning message. 2. Add restore data to LBA 0x55AA, if SMI vendor command fail. 3. Refine onButtonScanusb function 4. Refine check FlashID config on pretest and prescan. 5. In flash management, add check flash configuration |  |  |
| M0906A |  | sm22XMPToolM0906A.exe |
| 1. Remove all Teseter vendor command about Identify Device(0xEC). 2.Remove vendor command (F0/26) on button scanusb. 3. add SCSI command DoTestUnitReady() after reset device. 4.Modify Vendor command ReadFlash(F0/0A), page address hibyte is B7(Targ5),lowbyte is B4(Targ2) 5.remove Debug mode IDC_EDIT7, it does't used. 6. Fix Write Flash and Write block with SM2246 on debug mode. 7.Support Page 512 Pagetrans on Debugmode. |  |  |
| M0918A |  | sm22XMPToolM0918A.exe |
| 1.add function "D fail ECC" on debug mode 2.refine the code abour display status on initial flow. 3. add QC error code on QC procedure. 4. Add "Chunk ECC" function on debug mode. 5.Disable SM333 Tester Power off/On vendor command, it only does F0/2C Reset CPU vendor command. It only for SM333 Tester 8 port chorus issue. 6.Add “DRAM Vendor” option on parameter page(0:Nanya, 1:Winbond, 2: Samsung, 3: Hynix) 7.For Micron M73A(SLC), the RRTB setting as 99, it means that didn’t need RRTB setting. 8. add/Enable LTS info function for SM2246. |  |  |
| M1001A |  | sm22XMPToolM1001A.exe |
| 1. add QC setting option for individual QC item 2. add DRAM clock setting for SM2246 3. onbuttonscanusb , add read retry procedure if handle fail. |  |  |
| M1003A |  | sm22XMPToolM1003A.exe |
| 1. add CID option "RDT loops" and "Update RDT Bad Block Result" |  |  |
| M1009B |  | sm22XMPToolM1009B.exe |
| 1. Add button "save RDT data" on self test result tag page. 2. Fixed SN number issue when KeepSN is not 20 byte length. |  |  |
| M1024A |  | sm22XMPToolM1024A.exe |
| 1. Add 5 CID option "TranscendTSBTek", "AHCCMDTCompatible", "DCO","SCT","PHYEvent" 2. Add CID option, "Erase Count threshold" and "Spr Bk number threshold" for "TSB Tek" 3. Modify SM2246 "DumpOrigBad" and "RDT data" 4. Modift "Update Tester" function 5. Add function for Aptos RDT initail flow (FT1/FT2/FT3/EQA) 6. Add QC mode for Kingmax. 7. Update Aptos MP flow for SM2246 8. Add QC suberror code display on error code define button. 9. add QC item check for Netcom request |  |  |