慧榮科技股份有限公司

研發設計/變更版本管制表單				單版次: A

| 產品名稱: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 用途: 修改功能 |  |  |  |  |  |  |  |  |  |
| 隸屬專案: Zoom series |  |  |  |  |  |  |  |  |  |
| 此版本為: □新發行 ■設計變更 |  |  |  |  | 日期: 03/28/2014 |  |  |  |  |
| 資料類別歸屬: □IC H/W ■IC F/W □系統H/W □系統 F/W ■應用程式 ■其他 |  |  |  |  |  |  |  |  |  |
| 版次: | #### H/W版次 |  | F/W版次 |  |  |  | S/W版次 |  | Check Sum |
|  |  |  | ISP2246 N0328A |  |  |  | N0325A |  | 0x010A3BD0 |
| 內容概述: F/W: ISP: 1. Re-structure H2F mapping table. (for 1TB capacity support) 2. Extend the size of valid page count and unit page per block to 32bit. (for 4Ch-8Way 512Page and 1TB capacity support) 3. The address of dram table fluctuate according to the disk configuration. 4. Add dual dram setting. 5. Support the entry of device sleep without slumber mode.(CID 0x4D.bit7) BootISP: 1. Add dual dram setting. S/W: 1.add Show all RDT result button on 8Port RDT tagPage 2.add Compare SN function on BarCode dialog. 3.Fix Dump Erase count issue for TLC NF. 4.Add 0xF1/0x28 vendor command on DRAM Test procedure. 5. Check SN wheather is reaptly on Barcode dialog 6.Add SLC Region Capacity on CID setting for TLC. 7.Add new case "A-Top" with in DRAM Vendor fields, it is the same as "NANYA" setting, PM Stanley request. 8.Modify Verify Card Size rule for SM2246 TLC, RD David request 9.Fixed 8Port RDT Result Display. 10.If SN repetition, modify the Warning message box to "Continue" and "Exit" 11. If it is 0MB after scan drive, it will show"Can't Detect Drive" 12. Add "SaveSN WWNfail". 13. 8Port RDT repaint when change Tag page or screen move. 14. Add 4th fail case when RDT fail, "0xFF Pattern" 15. Mofied Time stamp units as "Secs" on MarkBad TagPage. 16. Scan Drive first when open MP tool, in PNY mode. 17. Modified 8 Port RDT display. 18. On BarCode, add "check Whether is 8 SN inputting" and "Check whether is any SN exist" 19. Modify message button text as "Continues" and "Return" 20. Remove Check PNY SN format on QC procedure. 21. On Barcode, add Warning message box for "Continues" and "Return". 22. When enable Barcode, if some Port, it didn't input SN, it will show warnong messsage "Port # Serial Number is empty in Barcode!"" 23. ON Scan Drive, if it didn't input SN on Barcode, it will show Warning message. 24. After push "finish" in Barcode, it will scan drive again automatically. 25. Mark Check PNY SN format. 26. Add Tag page "Assembly Verification" for PNY, in setting.set [OPTION] ENABLEASSEMBLYVERIFICATIONPAGE=1 27. Add "Dual DRAM" option for Pretest option setting Flash DB: - Add below list to flash DB TH58TEG9DBHBAS9 TH58TEG8D2JBASC 2. K9PHGY8S5A(1.8V) update Driving setting 3.JS29F32B08JCME2 Update mSATA and NGFF Driving setting 4.MT29F256G08CKEDB (L84C)/MT29F64G08AJABA (M62B)" add to list and support SM2246 |  |  |  |  |  |  |  |  |  |
| 建議通知客戶群組 |  |  |  |  |  |  |  |  |  |
| 需知會相關人員 |  | Vincent, Aaron, Roger, Bassa |  |  |  |  |  |  |  |
| 部門主管 |  | 單位主管 |  | 設計/變更 |  | 功能驗證 |  | 經辦人 |  |

接前頁內容概述：

| Program 變更事項 | Controller | 檔名 |
| --- | --- | --- |
| M0524 |  |  |
| MPTool New Firmware ISP |  | sm22XMPToolM0520A.exe ISP2246.BIN |
| M0531 |  |  |
| MPTool New Firmware ISP |  | sm22XMPToolM0528A.exe ISP2246.BIN |
| M0607 |  |  |
| MPTool New Firmware ISP |  | sm22XMPToolM0607A.exe ISP2246.BIN |
| M0610 |  |  |
| MPTool New Firmware ISP |  | sm22XMPToolM0610A.exe ISP2246.BIN |
| M0618 |  |  |
| MPTool |  | sm22XMPToolM0610A.exe |
| M0621 |  |  |
| MPTool New Firmware ISP |  | sm22XMPToolM0618A.exe ISP2246.BIN |
| M0626 |  |  |
| MPTool New Firmware ISP |  | sm22XMPToolM0626A.exe ISP2246.BIN |
| M0628 |  |  |
| New Firmware ISP |  | ISP2246.BIN |
| M0703 |  |  |
| New Firmware ISP |  | ISP2246.BIN |
| M0709 |  |  |
| New Firmware ISP |  | ISP2246.BIN |
| M0726 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM0723A.exe |
| M0808 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM0808A.exe |
| M0809 |  |  |
| MPTool |  | sm22XMPToolM0808B.exe |
| M0823 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM0815A.exe |
| M0830 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM0829A.exe |
| M0906 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM0906A.exe |
| M0918 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM0918A.exe |
| M0926 |  |  |
| New Firmware ISP |  | ISP2246.BIN |
| M1003 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM1001A.exe |
| M1007 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM1003A.exe |
| M1011 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM1009B.exe |
| M1024 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM1024A.exe |
| M1028 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM1024A.exe |
| M1101 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM1031A.exe |
| M1115 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM1114A.exe |
| M1129 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM1129A.exe |
| M1206 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM1205A.exe |
| M1213 |  |  |
| New Firmware ISP |  | ISP2246.BIN |
| M1227 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolM1226A.exe |
| N0102 |  |  |
| New Firmware ISP |  | ISP2246.BIN |
| N0103 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolN0102A.exe |
| N0117 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolN0114A.exe |
| N0124 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolN0122B.exe |
| N0207 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolN0206A.exe |
| N0214 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolN0213A.exe |
| N0307 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolN0307A.exe |
| N0328 |  |  |
| New Firmware ISP MPTool |  | ISP2246.BIN sm22XMPToolN0325A.exe |

| FW 變更事項 | Controller | 檔名 |
| --- | --- | --- |
| M0524A |  | ISP2246.BIN |
| 1. Fix refresh time-out issue 2. Support Mobile Mark power consumption measurement 3. Fix BIOS detect fail of NB04(FUJITSU AH550) 4. Set SMART thermal report under 69 degree |  |  |
| M0531A |  | ISP2246.BIN |
| 1) Support Mobile Mark power consumption measurement 2) Compatibility issue a) NB42, NB57 and NB 53 are host’s compatibility issue, it can pass by disable “FW partial to slumber”. 3) Fix power-cycling issue found from Netcom 4) Fix ECC early move bug 5) Change DRAM setting 6) Change retry index definition, up to 20 |  |  |
| M0607A |  | ISP2246.BIN |
| 1. Support 4ch 8way flash config. 2. Fix a power-cycling Dram backup bug. 3. Remove an unreasonable debug code in read-retry. 4. Update flash retry ID. |  |  |
| M0610A |  | ISP2246.BIN |
| 1. Update dram power-mode resume setting.(Dram fail issue) 2. Adjust the SPOR backup criterion. (SMI SPOR issue) |  |  |
| M0621A |  | ISP2246.BIN |
| 1. Enable dram compensation 2. Add vendor smart cmd. 3. Fix background clean bug. |  |  |
| M0626A |  | ISP2246.BIN |
| 1. Change power mode DramSetting 2. Fix Smart Info bug. 3. Fix NetCom powercyling fail issue. 4. Fix SMI SPOR fail issue. |  |  |
| M0628A |  | ISP2246.BIN |
| 1. Enhance command latency. 2. Enhance IO-meter IOPS. 3. Enhance CrystalDisk 512k read. 4. Fix a potential risk in standby mode. (VCT S4 BlueScreen issue- FWM0626A) 5. Fix a 4plane read-retry bug. 6. Fix a relink bug(Netcome power-cycling issue). |  |  |
| M0703A |  | ISP2246.BIN |
| 1. Fix bugs found in M0628A. 2. Reduce cached dram data from 16MB to 2MB. 3. Update SMART command ID. 4. Fix trim bug. 5. Fix Sata Error handle bug. |  |  |
| M0709A |  | ISP2246.BIN |
| 1. Adjust IPM time out threshold to improve pcmark score. 2. Fix SPOR miss-compare issue. 3. Fix swap bad block bug. 4. Fix trim bug. |  |  |
| M0726A |  | ISP2246.BIN |
| 1. Set flash IO to EDO mode for Toshiba & Sandisk flash read-retry sequence. 2. Modify Sata-Err-handle criteria. 3. Switch to dummy write if pure spare decrease to zero, and do not mark-bad anymore. 4. Disable auto D2H when reset cpu. 5. Modify SMART command. 6. Extend write log page to 32. 7. Fix strong page size bug. 8. Remove flash under driving strength setting iff Samsung & Toshiba flash. 9. Set timer interrupt from 12 second to 10 second. |  |  |
| M0808A |  | ISP2246.BIN |
| 1. Update smart command info. 2. Add JTAG wake up cpu function in IPM and standby mode. 3. Fix pretest bug for manual toggle flash. 4. Reset max correctable error count in ChkBlockRestECCFail() 5. Add gFlashDrive variable in parameter page for adjusting flash driving. 6. Add sata generation and cache program option in CID. 7. Add RDT function. 8. Modify IPM dram setting. |  |  |
| M0815A |  | ISP2246.BIN |
| 1. Increase dram clock from 330Mhz to 400Mhz 2. Add RDT, PreScan, and RestoreBad function. 3. Toshiba A19nm(A2 cmd disable so far) and Intel 20nm support. 4. Strong page only for whole drive support. 5. Further overdrive L85A flash IO. |  |  |
| M0830A |  | ISP2246.BIN |
| 1. Increase dram clock from 330Mhz to 400Mhz 2. Add RDT, PreScan, and RestoreBad function. 3. Toshiba A19nm(A2 cmd disable so far) and Intel 20nm support. 4. Strong page only for whole drive support. 5. Further overdrive L85A flash IO. |  |  |
| M0906A |  | ISP2246.BIN |
| 1.Add Samsung27nm MLC retry table. 2.Modify IM20nm retry policy, if the spare data become 0xFF, then switch to the next Vth. (M0830A L84 SPOR fail issue) 3.Fix build-link bug.(only occur on ch8way) (M0830A TSB19 4ch8way Netcom SPOR issue) - Do background task iff EC command finished. - Wait flash command queue down before resetting channel in sata error handle. - Fix GetBlockSerial function bug.(M0830A SS21nm SPOR issue) |  |  |
| M0918A |  | ISP2246.BIN |
| 1. Disable dram compensation.(KingMax burning issue) 2. Fix a wearleveing bug. (M0906A VCT SD-Stress issue) 3. Fix seek, read verify sector, and RW multiple command bug. 4. Fix a build F2H bug for act1 block which is also a powe-cycling tail block. (M0906A VCT SMI SPOR issue) 5. Backup all the end page of active block for power-cycling.(Netcom abnormal markbad issue) 6. Modify Micron 20nm(L84, L85) read retry set from 4 to 8. 7. Issue flash reset command before retry sequence. 8. Skip program dummy weak page for IM20nm flash. 9. Modify VDT40 power fail policy, While(1) iff VDT40 &VDT27 are equally true. 10. Add Toshiba A19nm SLC-mode block read-retry. 11. Lower schmitt trigger windows (level = 1) for 8CE configuration.( Transcend 8CE 2.5’’ demo board issue) 12. Support Samsung & Hynix dram. 13. Support LTS. 14. Update RDT function. 15. Add a vendor command for flash chunk read. |  |  |
| M0926A |  | ISP2246.BIN |
| High Priority: 1. Reset AHB before clearing dpp in DPS interrupt, and set-clear TSB buffer flag in sata error handler.(VCT M0918A issue item3,19 and Q寶 S4 issue). 2. Don’t reset flash in DisableReadRetry().(Hynix flash don’t accept this action before random data out) 3. Force re-build all the F2H of tail block whose EOB was ecc failed. (M0918A SMI SPOR issue). 4. Fix a power-cycling bug about backup the tail page of active block. 5. Change to new seed table. 6. Erase one empty map block per time in GenNextMapFBlock() to avoid power-cycling issue. 7. Reset the valid page count as 0xFFFF for bad block in relink. (already marked bad but haven’t save map) Low Priority: 8. Speed up SDB flow 9. Fix NCQ Write FUA bug. 10. Add Write Multiple FUA and Write DMA FUA 11. Support write cache enable/disable. 12. Fix VDT check bug. 13. Set bus div1 for 4CE and 8CE. 14. Change g16PushWriteTH for 4x8 config. 15. Issue multi-plane ALE instead of single-plane ALE in read-retry. 16. Add g32GRTPECycle. |  |  |
| M1003A |  | ISP2246.BIN |
| Low Priority: 1. Fix sata-error handle bug. 2. Add Micron20nm SLC read-retry table. 3. Add Dram 380Mhz option. 4. Fix DCO bug. 5. Add RDT loop function. 6. Enable write cache as a default. 7. Fix an A2 read bug for one plane flash. 8. Modify dram VDT from 1.4 to 1.3v. 9. Fix read_verify_sector &Seek command bug. 10. Support DMA setup auto activate. |  |  |
| M1003A |  | ISP2246.BIN |
| Low Priority: 1. Fix RDT bug (only update ISP2246SelfTest.bin) |  |  |
| M1003B |  | ISP2246.BIN |
| 1.Set Schmitt trigger window as 1 for DQS, ALE, CLE, WE, RE, and data pin. 2.Update RDT info. |  |  |
| M1011A |  | ISP2246.BIN |
| 1.Modify flash “Dll Adj” training method. 2.Passing Dell 1.6 script item except “DST Long continues after power-on”. 3.Disable all flash CE if channel is idle. 4.Do not finish partial clean in trim function for passing Win8 WHCK, if hit this situation we need save map before erasing any empty block. 5.Fix a power-cycling bug about backup the tail page of active block. 6.Add Command Support Option(CID 0x4c ) |  |  |
| M1024A |  | ISP2246.BIN |
| 1. Fix flush cache & HDMA over assignment bug in write flow. 2. Support SCT write same function. 3. Keep the empty blocks which is the source of the active block non-complete word-line pages to avoid data lose when power-cycling. 4. Fix Lite-on script issue. 5. Support “31” command for Netcom. 6. Exit background clean if any host command in except swapping ecc fail block to avoid command timeout. 7. Shut down flash core power in device sleep mode. 8. Support scale-down external-interleave way. |  |  |
| M1024B |  | ISP2246.BIN |
| 1. Set CID 0x4C.b7 = 0 to disable FW option to avoid power-cycling bug |  |  |
| M1101A |  | ISP2246.BIN |
| - Fix seek & read verify sector command bug. - Fix trigger write command bug of DMA and PIO. - Support PIO multiple mode to 2. - Fix SCT write same bug. - Modify sata RXTS from 512Bytes to 256Bytes for write performance. - Skip pre-read if lookahead was disabled. |  |  |
| M1115A |  | ISP2246.BIN |
| 1. No reset AHB in SataErrorHandle. (Ulink script) 2. Modify sata RXTS from 256Bytes to 512Bytes. (Ulink script) 3. Fix AddBufIdx bug. 4. Fix protect pair page algorithm bug. (over-program issue) 5. Fix 48 bit NonRw Command bug(LiteOn script) 6. Fix Write log Multi Sector count fail (LiteOn script) 7. Build first and last zone in re-link flow, and remove partial rebuild map in background idle condition. (WHCK) 8. Read parameter page from one info block to the other in swapInfoblock. 9. Reduce max trim sector count from 256 to 8. (WHCK) 10. Change disk’s default capacity. (no minus one block sector count) 11. Add SMART Attributes for LiteOn. 12. Reset Attributes by vendor command. 13. Disable SMART offline data collection capability bit 2(abort/restart by host) (LiteOn script) 14. Add Speed up Pret flow. 15. Add Support DevSleep option. 16. Add Support WWN option. |  |  |
| M1129A |  | ISP2246.BIN |
| 1. Fix wearleveling bug.( VCT burning issue) 2. Skip early move judgment if the retry page is last. (Micron IO-meter performance drop issue) 3. Fix a sata error handle bug. 4. Issue single-plane ALE instead of multi-plane for Intel and Micron read retry. 5. Force program finish page in “standby immediate”. 6. Add SMART command info requested from Kingmax. 7. Fix SMART attribute (0xc3) value miss-match bug. 8. Combine orphan block in RDT (Optional). 9. Add Micron 16nm read retry. 10. Add Samsung 19nm read-retry. BootISP: 11. Save parameter page index in garBootISPFlashInfo[3] for BootISP searching parameter page. |  |  |
| M1206A |  | ISP2246.BIN |
| 1. Fix Trim bug.(M1128A VCT burning issue) 2. Support download micro code. 3. Strong page only for Write Protect block. 4. Add read retry iteration option in CID. 5. Add a CID option for toggling DASP when RW command in. 6. Add a CID option for disabling SSC. 7. Add index version in Index block spare for firmware compatibility. 8. Re-check disk capacity in RDT. |  |  |
| M1213A |  | ISP2246.BIN |
| 1. Fix UNC bug. 2. Fix Ferri build link bug. 3. Fix read boot ISP bug in mark bad. 4. Fix WHCK trim performance issue. 5. Temporarily disable “protect pair page” function. |  |  |
| M1227A |  | ISP2246.BIN |
| 1. Fix parameter page information miss-match bug. 2. Support combine bad block with plane base. 3. Support single die mark bad, and use rest good block to re-combine super block. 4. Modify early move policy. 5. Do not search parameter page in ISP to speed up boot time. 6. Modify RDT test rule. 7. Support DMA download microcode, and fix bugs. 8. Return status 0x50 for Trim LBA(0xFFFF) and transferring count(0xFFFF). 9. Correct some SMART info. 10. Add SMART UNC test function. 11. Disable IPM before reset CPU. |  |  |
| M1213C |  | ISP2246.BIN |
| - Fix bad block count miss-match bug. - Fix swap power-cycled active block bug. - Adjust swap block priority. - Disable IPM before reset cpu. |  |  |
| N0103A |  | ISP2246.BIN |
| 1. Fix swap power-cycled active block bug. 2. Adjust swap block priority. (Clean higher than swap ecc block) 3. Add a parameter variable, called g16MarkBadCnt for counting mark bad block count. 4. Modify bank number definition from 1024 to 512 blocks. 5. Fix a potential command queue over flow risk in InitReadSource(). 6. Re-combine orphan block table if “reference runtime bad” for Pret. 7. Add full dram size test in pretest. 8. Add customized “World Wide Name” for ID table. 9. Force save map if unsaved block count over 32. 10. Flush D-Cache before vendor read & write ram command. 11. Support L84C. 12. Don’t do RDT if first power-on. |  |  |
| N0103B |  | ISP2246.BIN |
| 1. Use toggle mode interface instead of EDO mode to set Samsung 19nm read retry sequence. 2. Modify SMART erase count threshold from 50 to 0. 3. Support DMA read log extend and DMA write log extend. 4. Support DMA read buffer and DMA write buffer. |  |  |
| N0124A |  | ISP2246.BIN |
| 1. Speed up boot up time by saving spare bitmap table, and shortening mapping table reset time. 2. Shorten the first read command response time of device sleep mode. 3. Re-issue flash multi-plane ALE after disabling read-retry. 4. Issue one plane ALE instead of multi-plane in read-retry. 5. Keep the first 8k byte of ISP code if doing download microcode. (Lite-on) 6. Keep RDT info in info block. |  |  |
| N0207A |  | ISP2246.BIN |
| 1. Extend the write pulse width for setting read-retry sequence in EDO mode. 2. Enable hardware write protect.(GPIO p1.bit1) 3. Enable quick erase.(GPIO P1.bit5) 4. Fix the SMART value (attribute ID 0x05) miss-match issue. |  |  |
| N0214A |  | ISP2246.BIN |
| 1. Extend the ALE CLE input pulse width for setting read-retry sequence in EDO mode. 2. Fix a fast boot bug. 3. Fix a partial move factor calculation bug.(This bug will only affect the command latency on 8k flash). 4. Return error for a write command when pure spare become zero. (Trig dummy write in the previous version). 5. Modify sata TX impedance value(100Ω). 6. Modify sata III TX pre-emphasis value from 0x14 to 0x08. 6. Modify sata RX equalizer value from 0x02 to 0x01. 7. Claim the required time for security erase with different capacity in ID table. |  |  |
| N0307A |  | ISP2246.BIN |
| 1. Fix a markbad bug. (N0227A issue from VCT) 2. Fix SMART info miss match issue. 3. Fix read and write log DMA extend command bug. 4. Fix RDT firmware cannot recognize MP vendor command issue. 5. Support 16k 4plane flash. 6. Modify the host read and write count unit from 32MB to 1GB for PNY SMART info. |  |  |
| N0328A |  | ISP2246.BIN |
| ISP: 1. Re-structure H2F mapping table. (for 1TB capacity support) 2. Extend the size of valid page count and unit page per block to 32bit. (for 4Ch-8Way 512Page and 1TB capacity support) 3. The address of dram table fluctuate according to the disk configuration. 4. Add dual dram setting. 5. Support the entry of device sleep without slumber mode.(CID 0x4D.bit7) BootISP: 1. Add dual dram setting. |  |  |

| SW 變更事項 | Controller | 檔名 |
| --- | --- | --- |
| M0528A |  | sm22XMPToolM0528A.exe |
| 1.Seperate SM2244 and SM2246 Display window individually. 2. Add SM2246 CID option [0x01] bit6 Partial2Slumber. 3. Add flash ID to Pretest header 0x3A~0x3F from flash DB. |  |  |
| M0607A |  | sm22XMPToolM0607A.exe |
| 1. Fix AutoTest for SM2246 2. Disable DRAM file setting for SM2246(Current SM2246 didn't need DRAM setting file,Aaron request) 3. Add FormFactor combo select item for SM2246 Driving setting using. 4. add 4 kinds of Driving setting to CID driving setting for SM2246. 5. Add check procedure on flash CE number for SM2246. 6. Fix Show TagPage flow 7. In SM2246 Flash DB, divide driving setting into four kinds(a.EVB, b. 2.5Inch, c.mSATA , d.NGFF) |  |  |
| M0610A |  | sm22XMPToolM0610A.exe |
| 1. Fixed flow of check CE number within all channels on pretest procedure |  |  |
| M0618A |  | sm22XMPToolM0618A.exe |
| 1. Fix Flash ID judgment method. 2. when do vendor command F1/28 extend timeout. 3. Modify "F_opt2" as 2 digits for new option. 4. Fix "read retry flash id" fail. |  |  |
| M0626A |  | sm22XMPToolM0626A.exe |
| 1. Add flow of check Read Flash ID to fix that vendor command interrupt. |  |  |
| M0723A |  | sm22XMPToolM0723A.exe |
| 1. Fix read Global page function on debug mode 2. Add check Sum function , in setting.set [OPTION] ENABLECOMPCHKSUM=1 3. Fix Pretest button on debug mode, it can download MPISP for SM2246. 4. Update CID setting display 5. Modify Strong page table flow (256=>1024 bytes) |  |  |
| M0808A |  | sm22XMPToolM0808A.exe |
| 1. add show Pretest header in CID option on DialogReadCID. 2. add new flash database item, flash output driving for SM2246 3. add Max SATA speed CID option 4. add DRAM test for SM2246 5. add do prescan on SM2246, in normal initial procedure and in page diagnosis "Bad Block (Original) Diagnosis". 6. show SelfTest page in SM2246 7. add En cache program CID option byte1 bit1 8. compare with FlashID(F0/20) and ID table when verify card size |  |  |
| M0808B |  | sm22XMPToolM0808B.exe |
| 1. modify verify card size,compare totla data sector with FlashID buffer(F0/20) and ID table Word 100~103 |  |  |
| M0815A |  | sm22XMPToolM0815A.exe |
| 1.Add "Other setting" button for extra CID setting page using on parameter page. 2.Add new CID setting, adjust temperature. 3.Update ReadCID page display. 4.Remove not using setting on parameter page. |  |  |
| M0829A |  | sm22XMPToolM0829A.exe |
| 1. Fixed the code about warning message. 2. Add restore data to LBA 0x55AA, if SMI vendor command fail. 3. Refine onButtonScanusb function 4. Refine check FlashID config on pretest and prescan. 5. In flash management, add check flash configuration |  |  |
| M0906A |  | sm22XMPToolM0906A.exe |
| 1. Remove all Teseter vendor command about Identify Device(0xEC). 2.Remove vendor command (F0/26) on button scanusb. 3. add SCSI command DoTestUnitReady() after reset device. 4.Modify Vendor command ReadFlash(F0/0A), page address hibyte is B7(Targ5),lowbyte is B4(Targ2) 5.remove Debug mode IDC_EDIT7, it does't used. 6. Fix Write Flash and Write block with SM2246 on debug mode. 7.Support Page 512 Pagetrans on Debugmode. |  |  |
| M0918A |  | sm22XMPToolM0918A.exe |
| 1.add function "D fail ECC" on debug mode 2.refine the code abour display status on initial flow. 3. add QC error code on QC procedure. 4. Add "Chunk ECC" function on debug mode. 5.Disable SM333 Tester Power off/On vendor command, it only does F0/2C Reset CPU vendor command. It only for SM333 Tester 8 port chorus issue. 6.Add “DRAM Vendor” option on parameter page(0:Nanya, 1:Winbond, 2: Samsung, 3: Hynix) 7.For Micron M73A(SLC), the RRTB setting as 99, it means that didn’t need RRTB setting. 8. add/Enable LTS info function for SM2246. |  |  |
| M1001A |  | sm22XMPToolM1001A.exe |
| 1. add QC setting option for individual QC item 2. add DRAM clock setting for SM2246 3. onbuttonscanusb , add read retry procedure if handle fail. |  |  |
| M1003A |  | sm22XMPToolM1003A.exe |
| 1. add CID option "RDT loops" and "Update RDT Bad Block Result" |  |  |
| M1009B |  | sm22XMPToolM1009B.exe |
| 1. Add button "save RDT data" on self test result tag page. 2. Fixed SN number issue when KeepSN is not 20 byte length. |  |  |
| M1024A |  | sm22XMPToolM1024A.exe |
| 1. Add 5 CID option "TranscendTSBTek", "AHCCMDTCompatible", "DCO","SCT","PHYEvent" 2. Add CID option, "Erase Count threshold" and "Spr Bk number threshold" for "TSB Tek" 3. Modify SM2246 "DumpOrigBad" and "RDT data" 4. Modift "Update Tester" function 5. Add function for Aptos RDT initail flow (FT1/FT2/FT3/EQA) 6. Add QC mode for Kingmax. 7. Update Aptos MP flow for SM2246 8. Add QC suberror code display on error code define button. 9. add QC item check for Netcom request |  |  |
| M1031A |  | sm22XMPToolM1031A.exe |
| - Add CID option: command31. - Fixed RDT/LTS display repeat. - Add QC item. DRAM size/Vendor/setting/clock. |  |  |
| M1114A |  | sm22XMPToolM1114A.exe |
| 1. Fix display Original bad for SM2246 on selftest page 2. Add option "Speed Up Pretest" and "RDT ECC Threshold" 3. add CID option "Device Sleep mode" and "world Wide Name" 4. On debug mode , add change SMART value. 5. Add Netcom QC mode option on setting.set 6. add RDT error code on selftest page. 7. add showing Original bad on RDT result. 8. Save RDT result to binary file. 9. Fixed not check “enable smart” 10. It will display all CID fail specific code 11. Remove warning message when load old version ini file 12. Add required administrator privilege automatically when running on WinVista/Win7/Win8 13. Add check ISP file, more check, pretest channel/ce, strong page ratio and new CID option in QC setting page. 14. Add notice string to key in decimal when turning on “set capacity manually” 15. Fix card mode display error, the card mode setting of FW is based on FW M1003 and document M1003. 16. add CID option "Disable SMART Temperature" 17. Aptos add save RDT data checkbox 18. Save As Config modification 19. Setting.set Add SN16=1 to support hex in SN |  |  |
| M1129A |  | sm22XMPToolM1129A.exe |
| 1. add SN Indicator function for Transcend. 2. Update ID table WORD 80 fields string. 3. marked A2 command flag when read Info page,for read Strong page only . |  |  |
| M1205A |  | sm22XMPToolM1205A.exe |
| 1. Add "create file" for linux tool on setting page 2. add CID option SMART Vendor ID on offset 0x4F 3. Fixed SN Indicator function 4. Disable Dialog MAXIMIZEBOX button (ShiuanWei Request) 5. Add CID option "Toggle DASP","Disable SSC",and "Read Retry iteration" |  |  |
| M1226A |  | sm22XMPToolM1226A.exe |
| 1. Modify Bad block count caculation by plane. 2. Add new option for Bar code SN function , option EnableBarCodeKeep 3. Fix the UI alignment. 4. add new CID option "Early Retire PE" 5. Remove pretest CH/CE item "0" on setting page. 6. Early Move TH and Early Retire THitem increasing to 66 |  |  |
| N0102A |  | sm22XMPToolN0102A.exe |
| 1.Add WWN setting on ID Tablw Word 108~111 2.Add DRAM Test Loop on setting page 3.Add column "Plane" on MarkBad Tag page. |  |  |
| N0114A |  | sm22XMPToolN0114A.exe |
| 1. Add TLC WAI on diagnosis page, save erase count info 2. Add Pretest fail case => Adjust Training fail flash ID byte 1 is 0x50 3. In Selftest page, modify the criterion of RDT fail, it only refer to the RDT info offset 0x0F error code 4. Add the IDEMA capacity on setting page with setting capacity manually. 5. Add option "Adj. Trainning Fail" 6. Add the SN indicator function with SM2246, Only modify the ISP. 7. Add WWN 16 byte from WWN.ini |  |  |
| N0122B |  | sm22XMPToolN0122B.exe |
| 1. on debug mode, add TLC compare and read data, interleave setting, add TLC program mode. 2. Fixed the bug for auto select flash on diagnosis page 3.Add Transcend Swap ISP mode option, for swap ISP only. 4. add compare TLC function within debug mode 5. Add button with display all CH/CE Flash ID on CIDDialog page. 6. Update ID table item for JJ request on ID table tagPage. 7. Modify SN minimum length as 12, original is 13. 8. add Port remapping function. 9. add PNY WWN feature. 10. add special SWAP ISP function for Transcend. 11. Display Tester FW version after updating Tester |  |  |
| N0206A |  | sm22XMPToolN0206A.exe |
| 1. add PNY format WWN ,Setting.set [FUNCTION] ENABLEPNYWWN=1 2. add option 8Port RDT result tag page, In Setting.set [OPTION] ENABLE8PORTRDTTAGPAGE=1 3. add PNY mode to disable WWN edit and take off Disk capacity 512MB ~ 2048GB in setting page,Setting.set [OPTION] ENABLEPNYMODE=1 4. add check Input SN format in Barcode dialog. 5. for PNY, RDT hours and loops as 1~255. 6. enable CID option Quick Erase and hardware write protect for SM2246 7. fix PNY WWN format fail. |  |  |
| N0213A |  | sm22XMPToolN0213A.exe |
| 1. add Internal interleave mode to increase CE number when download ISP 2. add check inputting SN format when enable Bar code 3. add option for only show main page and 8 port RDT result page 4. add PNY mode for PNY option, if pretest bad performance, it will return fail error code. |  |  |
| N0307A |  | sm22XMPToolN0307A.exe |
| 1. Fix no display "No check Flash ID" on setting page. 2. Add "Save Flash ID to log File" if it compare Flash ID fail when initializing card. 3. Modify Barcode flow for PNY request when enanle PNY mode. 4. Add capacity option "48GB" IDEMA mode for PNY mode. 5. Add SN Mask case "WW" on setting page, the week number this year for system time. 6. modify Bar code flow for PNY request when enable PNY mode. 7. Modify "Save EC" on debug mode and diagnosis tagPage for TLC NF case. 8. Add check SN and WWN which PNY format on QC procedure . 9. Add new Pretest error case "flashID[0x01]=0x06, show “Need Reduce Ext-IntlvWay" |  |  |
| N0325A |  | sm22XMPToolN0325A.exe |
| 1.add Show all RDT result button on 8Port RDT tagPage 2.add Compare SN function on BarCode dialog. 3.Fix Dump Erase count issue for TLC NF. 4.Add 0xF1/0x28 vendor command on DRAM Test procedure. 5. Check SN wheather is reaptly on Barcode dialog 6.Add SLC Region Capacity on CID setting for TLC. 7.Add new case "A-Top" with in DRAM Vendor fields, it is the same as "NANYA" setting, PM Stanley request. 8.Modify Verify Card Size rule for SM2246 TLC, RD David request 9.Fixed 8Port RDT Result Display. 10.If SN repetition, modify the Warning message box to "Continue" and "Exit" 11. If it is 0MB after scan drive, it will show"Can't Detect Drive" 12. Add "SaveSN WWNfail". 13. 8Port RDT repaint when change Tag page or screen move. 14. Add 4th fail case when RDT fail, "0xFF Pattern" 15. Mofied Time stamp units as "Secs" on MarkBad TagPage. 16. Scan Drive first when open MP tool, in PNY mode. 17. Modified 8 Port RDT display. 18. On BarCode, add "check Whether is 8 SN inputting" and "Check whether is any SN exist" 19. Modify message button text as "Continues" and "Return" 20. Remove Check PNY SN format on QC procedure. 21. On Barcode, add Warning message box for "Continues" and "Return". 22. When enable Barcode, if some Port, it didn't input SN, it will show warnong messsage "Port # Serial Number is empty in Barcode!"" 23. ON Scan Drive, if it didn't input SN on Barcode, it will show Warning message. 24. After push "finish" in Barcode, it will scan drive again automatically. 25. Mark Check PNY SN format. 26. Add Tag page "Assembly Verification" for PNY, in setting.set [OPTION] ENABLEASSEMBLYVERIFICATIONPAGE=1 27. Add "Dual DRAM" option for Pretest option setting 28. Add display RDT Hours and Loops in "8port RDT result" tag page. 29. For PNY, it will show QList on initial MP tool. |  |  |