慧榮科技股份有限公司

研發設計/變更版本管制表單				單版次:A

| 產品名稱:Solo Tester / SM2246XT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 用途: Update FW |  |  |  |  |  |  |  |  |  |
| 隸屬專案: Zoom series |  |  |  |  |  |  |  |  |  |
| 此版本為: □新發行 ■設計變更 |  |  |  |  |  | 日期: 05/19/2014 |  |  |  |
| 資料類別歸屬: □IC H/W ■IC F/W □系統H/W □系統 F/W ■應用程式 □其他 |  |  |  |  |  |  |  |  |  |
| 版次: | #### H/W版次 |  | F/W版次 |  | S/W版次 |  |  | 其他 |  |
|  |  |  | N0519B |  | N0520A |  |  | Flash.SET |  |
| 內容概述： FW: Pretest: 1.modify calculation of HBlock -(Old)System reserve block : 8+WPRO(1)+ H2F Tab + Cache Info(1) + GC(2) +Qboot(1) (New)System reserve block : 8+WPRO(1)+ H2F Tab + Cache Info(1) -(Old)Spare Block : HBlock/(Spare Ratio) + 3 (New)Spare Block : HBlock/(Spare Ratio) 2.modify data sectors : -(Old) original table (New) 46En capacity sectors table 3.Pretest Error code condition -(New) if (PretestBadCnt > g16TotalFBlock*25+1023)/1024) -> TooMuchBad -(New) if (HBlock > Ideal request HBlock) -> LowerCapacity Those error codes are needing MP tool to stop it and show fail. SW: - Add Check Pretest Error Code C_Pret_LowerCapacity & C_Pret_TooMuchBad. - Fixed Capacity Function. - Add Spare Ratio Function. - Add Clear Card Function in Pretest switch. Others 1. Flash.SET Update N0521 |  |  |  |  |  |  |  |  |  |
| 建議通知客戶群組 |  |  |  |  |  |  |  |  |  |
| 需知會相關人員 |  |  |  |  |  |  |  |  |  |
| 部門主管 |  | 單位主管 |  | 設計/變更 |  |  | 功能驗證 |  | 經辦人 |

| F/W變更事項 |  |
| --- | --- |
| ISP code 版本 | 變更事項 |
| N0519B | Pretest: 1.modify calculation of HBlock -(Old)System reserve block : 8+WPRO(1)+ H2F Tab + Cache Info(1) + GC(2) +Qboot(1) (New)System reserve block : 8+WPRO(1)+ H2F Tab + Cache Info(1) -(Old)Spare Block : HBlock/(Spare Ratio) + 3 (New)Spare Block : HBlock/(Spare Ratio) 2.modify data sectors : -(Old) original table (New) 46En capacity sectors table 3.Pretest Error code condition -(New) if (PretestBadCnt > g16TotalFBlock*25+1023)/1024) -> TooMuchBad -(New) if (HBlock > Ideal request HBlock) -> LowerCapacity Those error codes are needing MP tool to stop it and show fail. |
| N0519A | ISP: 1.No need to do refresh at SLC flash. 2.Break clean child when Sata Cmd In. 3.Wait Cmd Fifo ready after reset flash. MPISP: 1. Add rebuSysBlk 2. Fix ReadPar & ReadFlashID Pretest: 1. Program dummy data to info: change close block to close wordline. |
| N0509B | Pretest: 1. Enable DLL adjustment. 2. Improve pretest time. |
| N0509A | 1. Fixed N0506A S3/S4 BSOD Issue. |
| N0506A | Pretest: 1. Disable DLL adjust function. ISP: 1.Fixed the “Reset Security Mode” clean TSB0 in the ISR. 2.Using chkChgH2fSrcBlock() to consider more case instead of using cmprDataSerial(). 3.Recovered uspRdSrcAddr variable at end of doReadRetry function to prevent mis-judge of ECC Error(Refresh ECC error page). 4.Limit GC block number when delay return status for compatibility 5.Get correct buffer ptr when data transfer be interrupted by ESD test. 6.To make sure any Sata Error before data transfer completely. |
| N0423A | 1. Fix S4 issue. Do not refresh the same block again. 2. Modify the read bad info vendor command. |
| N0422A | - Fix L83A SPOR mark bad issue. - Fix HPA “Set Max Address Ext” bug. |
| N0421A | 1.Fixed bug : for SSP Test1 “Initial Device Parameters” test fail. |
| N0407A | PRJ_BOOT 1. Enable refresh function. 2. Enable child group number limited when rebuspare block. 3. Modify the GcInfo TH, and pushQ TH. PRJ_ISP 1. Modify chkPushSpareQ(). The block of VPcnt=0 push to spare first. 2. Disable SKIPGCSRCH. |
| N0403A | 1. Fixed the SMART ID (0x09, Power On Hours) loss after reboot. |
| N0401A | Fixed bug: - Disable rdlink refresh function. - Disable child group number limit when rebuspare block. - ID Table : Word 7(LSB)-Word8(MSB) --> Word 7(MSB)-Word8(LSB) - WPRO Block : Read SMART Table fail. - ISP_RdlinkFunc.c : don't mark Wpro block erase cnt = 0xFFFF Add: - Do chkPushSapreQ() after chkBgdClnH2fTabBlk() to prevent from FW run out of spare blocks. - Add Power-on refresh data flow - Add small child group number TH in rebuSpareFblock() - Insert NOP in TSB24MLC Retry Command |
| N0324A | - Fix Vendor Command: Read ID Table - Report not enough capacity from pretest |
| N0317A | - Add support HPA feature set. - Add support ACTPM feature. - Update SMART: read error rate, power on hours. - Fix bug: set gabage collection priority higher than wearleveling in lower avaliable spare blocks case. |
| N0310A | - Redefine Child Block Number to prevent run out of spare block in low density configuration. |
| N0307A | - Fix SPOR test issue: FW will fail to detect SPOR damaged page data in some cases. - Change initial pattern from all ‘1’ to all ‘0’. - Fix out of range bug (Lexar found) in read flow. |
| N0227A | - add support of security feature set - fix N0219A test issues: - SPOR data comparing error: add protection to old data - 2) Burning issue: fix run out of spare block bug |
| N0219 | - Fix bug and improve power cycling test - Fix S3/4 timeout issue at L8 series NAND - Fix bug: SMART ‘minimum erase count’, ‘number of valid spare blocks’ - 4.Add option for setting SATA gen |
| N0218 | - Fix bug and improve power cycling test - Fix S3/4 timeout issue at L8 series NAND - Fix bug: SMART ‘minimum erase count’, ‘number of valid spare blocks’ - 4.Add option for setting SATA gen |
| N01231 | - 1.Add new RetryType for L95A and L84C MLC |
| N01221 | - Fix BSOD Error Code 0xA0: bug of a wrong FW flag. - Add timing mode and over driving setting for Micron/Intel Nand. - Fix some read retry bugs. |
| N01081 | - Fix a read retry bug in power up initial flow - Fix interleave read bug in power up initial flow |
| M12241 | - Add IM read retry flow - Fix bug: read flow bug caused burning failure |
| M12201 | - Fixed turn off dram pll bug. |
| M12191 | - Support M84C multi-die configuration. - Improve sequential read performance. - Fix ECC failure handling bug. - Adjust thermal sensor report value and add option to +/- |
| M11291 | - Add support Micron Flash M84C |
| M11251 | - Add read partial table to increase 4k random r/w - Add hdtune trick - Add background clean to improve dirty mode perf |
| M11151 | - Support Interleave Mode - Support SMART Function - SPOR bugs |
| M11062 | - Fix SLC/2-plane mode fail @ suspend tool issue - Improve SPOR protection |
| M11061 | - Disable V1.8 VDT |
| M1024 | - VDT23,VDT27,VDT40 protection - Support Toshiba 24nm SLC - Support Disable Interleave Mode |
| M09302 | - Only support Toshiba TC58TEG6DCJTA00 Flash - Add Write cache feature set (word 82 bit5) - Add LBA48 feature set (word 83 bit10) - Add HIPM (word 76 bit9) - Add DIPM (word 78 bit3) - Add Read PAR vendor command - Add Flash Control/DQS driving setting |
| M09171 | - First Version - Only support Toshiba TC58TEG6DCJTA00 Flash |

| Pretest code版本 | 變更事項 |
| --- | --- |
| M0917 | - First Version |

| MP ISP code版本 | 變更事項 |
| --- | --- |
| M0430 | - First Version |

| Boot ISP code版本 | 變更事項 |
| --- | --- |
| M0917 | - First Version |

| Prescan code版本 | 變更事項 |
| --- | --- |

| AP變更事項 |  |
| --- | --- |
| AP版本 | 變更事項 |
| N0520A | - Add Check Pretest Error Code C_Pret_LowerCapacity & C_Pret_TooMuchBad. - Fixed Capacity Function. - Add Spare Ratio Function. - Add Clear Card Function in Pretest switch. |
| N0514B | Fixed Read CID No SMI Tester Issue. |
| N0512A | 1. Enable Clock adjustment Info. |
| N0507C | 1. Fixed Load Data Base Extend Block Shift Issue(36). |
| N0505C | 1. Disable Clock adjust info. |
| N0505B | 1. Add Clock adjust info. 2. Add different address and Flash Clock in Pretest. |
| N0422B | 1. Update database L85A over driving 2.Fix Update Flash driving no update issue |
| N0422A | 1. improvement MP get no flash IO driving use FW default driving. |
| N0421A | 1.Update Data Base Flash IO Driving Setting. |
| N0407A | 1.Disable for 2236 Function |
| N0401A | - Fix scan tester issue |
| N0325A | - Fix scan tester issue |
| N0324A | - Check pretest result for capacity |
| N0317A | - Add show spare block - Add HPA option - Fix Display Card mode |
| N0311A | - Show spare block in CID page |
| N0304A | - Fix sk close MP tool fold. |
| N0227A | - Display ISP CheckSum - Add Display Card mode - Add security option. |
| N0122A | - setting SATA Bus speed |
| N0122A | - ate word 3 value of Identify depend on disk size |
| N0108A | - Add support NCQ option. - Add Wear Leaveling Gap option. (setting.set[OPTION] add ShowWearLevelGapOption=1 to display wear leveling gap option) - Add display current CID setting and raw data - Add preload option |
| M1224A | - Add support device sleep option. - Add manual flash clock option. (setting.set[OPTION] add ShowFlashClockOption=1 to display flash clock option on parameter page) |
| M1218A | - Setting.set [Option] SupportHotKey=1 -> support use keyboard to control MPTool. R/r key : scan device, Space key : initial card - Add support trim option - Add card size option(support auto, specific size or specific sectors) - Add adjust temperature option - Interleave option change to use (1) FW auto (2) disable 2way(force disable) (3) disable 4way(run 2way or disable) |
| M1125A | - Refine display of card size in the main page - FW version will display year-month-day instead of Mxxxx |
| M1115A | - Add CFast option - Add External Interleave option - Add display ISP file checksum in main page - Add format option - Fix flash setting is wrong if user doesn't auto flash and save |
| M1105A | - Add Copy Compare Function - Fix Select CE and Active Channel num wrong in debug mode |
| M1004A | - Add Flash Control/DQS driving setting |
| M0917A | First Release |