慧榮科技股份有限公司

研發設計/變更版本管制表單				單版次: A

| 產品名稱: |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 用途: 修改功能 |  |  |  |  |  |  |  |  |  |
| 隸屬專案: Zoom series |  |  |  |  |  |  |  |  |  |
| 此版本為: □新發行 ■設計變更 |  |  |  |  | 日期: 12/27/2011 |  |  |  |  |
| 資料類別歸屬: □IC H/W ■IC F/W □系統H/W □系統 F/W □應用程式 □其他 |  |  |  |  |  |  |  |  |  |
| 版次: | #### H/W版次 |  | F/W版次 |  |  |  | S/W版次 |  | 其他 |
|  |  |  | ISP2250new K1227A |  |  |  | K1222A |  |  |
| 內容概述: F/W: - Fix side effect of only one read/write command queue(bug in K1220A) - Fix hang-up problem in power mode(bug in K1212A) S/W: |  |  |  |  |  |  |  |  |  |
| 建議通知客戶群組 |  |  |  |  |  |  |  |  |  |
| 需知會相關人員 |  | Jacky Lin, Vincent,Aaron,Roger,Bassa |  |  |  |  |  |  |  |
| 部門主管 |  | 單位主管 |  | 設計/變更 |  | 功能驗證 |  | 經辦人 |  |

接前頁內容概述：

| Program 變更事項 | Controller | 檔名 |
| --- | --- | --- |
| K11224 |  |  |
| MPTool New Firmware ISP | SM2250AB | sm22XMPToolK1123.exe ISP2250new.BIN |
| K11224 |  |  |
| New Firmware ISP | SM2250AB | sm22XMPToolK1123.exe ISP2250new.BIN |
| K11228 |  |  |
| New Firmware ISP MPTool | SM2250AB | sm22XMPToolK1128.exe ISP2250new.BIN |
| K1129 |  |  |
| New Firmware ISP | SM2250AB | ISP2250new.BIN |
| K1130 |  |  |
| New Firmware ISP | SM2250AB | ISP2250new.BIN |
| K1202 |  |  |
| New Firmware ISP MPTool | SM2250AB | ISP2250new.BIN sm22XMPToolK1201B.exe |
| K1205 |  |  |
| New Firmware ISP MPTool | SM2250AB | ISP2250new.BIN sm22XMPToolK1204A.exe |
| K1208 |  |  |
| New Firmware ISP MPTool | SM2250AB | ISP2250new.BIN sm22XMPToolK1208A.exe |
| K1209 |  |  |
| New Firmware ISP | SM2250AB | ISP2250new.BIN |
| K1212 |  |  |
| New Firmware ISP MPTool | SM2250AB | ISP2250new.BIN sm22XMPToolK1209A.exe |
| K1216 |  |  |
| New Firmware ISP MPTool | SM2250AB | ISP2250new.BIN sm22XMPToolK1216A.exe |
| K1220 |  |  |
| New Firmware ISP MPTool | SM2250AB | ISP2250new.BIN sm22XMPToolK1220A.exe |
| K1221 |  |  |
| New Firmware ISP MPTool | SM2250AB | ISP2250new.BIN sm22XMPToolK1221A.exe |
| K1222 |  |  |
| New Firmware ISP | SM2250AB | ISP2250new.BIN |
| K1227 |  |  |
| New Firmware ISP | SM2250AB | ISP2250new.BIN |

| FW 變更事項 | Controller | 檔名 |
| --- | --- | --- |
| K1124A |  | ISP2250new.BIN |
| 1. PIO mode vendor command bug 2. Capacity less than 8GB in AHCI mode display problem. |  |  |
| K1124B |  | ISP2250new.BIN |
| 1.Return RW command error under security lock state (Aptos issue) |  |  |
| K1128A |  | ISP2250new.BIN |
| 1. Reset flash channel flow for Hynix DDR flash. |  |  |
| K1129A |  | ISP2250new.BIN |
| 1. HPA bug about set max address and read native max address |  |  |
| K1130A |  | ISP2250new.BIN |
| 1. fix Potential bug in power cycle |  |  |
| K1130B |  | ISP2250new.BIN |
| 1. fix Potential bug in power cycle |  |  |
| K1202A |  | ISP2250new.BIN |
| 1.Fix EC table and command supporting according to TSB’s suggestion 2.Fix bug about assign g16PretestBadCnt |  |  |
| K1202C |  | ISP2250new.BIN |
| 1.fixes the bug(AvgErCnt > MaxErCnt) from Transcend (Hsuan). |  |  |
| K1205A |  | ISP2250new.BIN |
| 1. Fix a bug power cycling bug (from Netcom) |  |  |
| K1208A |  | ISP2250new.BIN |
| 1. Update RDT function 2. Support TSB G9FBASA Flash 3. Fix Manual Capacity |  |  |
| K1209A |  | ISP2250new.BIN |
| 1.Fix the judgment about LBA continue of read/write command. |  |  |
| K1212A |  | ISP2250new.BIN |
| 1. Extend block erase count to 4B in RDT result. 2. FW workaround PIO sector count=65536 case (Toshiba issue item 4) 3. Implement check power mode command (Toshiba issue item 5) 4. Change SMART item definition, write/read sector use 32MB as based just like Intel’s SSD |  |  |
| K1216A |  | ISP2250new.BIN |
| 1. Fix bug about save SMART info in K1212A 2. Fix a power-cycle bug (g16arMapSavedLastActive is empty block) 3. Fix bug in SwapInfoBlock2() 4. Support all zero RDT result in normal ISP 5. Add thermal sensor option in OPT[0x00].b2, default=0 (27C) 6. CacheBack for PC Mark vantage item5 7. A little bit performance up a. Set S_DebugDram=0 and S_DebugDram = 0 b. Add g16FlushCacheDelay from 0xA00 to 0x30 |  |  |
| K1220A |  | ISP2250new.BIN |
| 1. Only one read/write command queue(TSB issue 2) 2. Delay send SATA signature @ CID[0x00].b3 to avoid EC command time-out for Transcend’s dedicated bridge 3. Enable early move/retire function by MP tool 4. Fix EC table word 1 as 0x0040 (TSB issue 13) 5. Recover gbCacheEnable if(!SSP)(TSB issue 12) 6. Fix EC table word 105 (Trim sec count) (TSB issue 17) |  |  |
| K1221A |  | ISP2250new.BIN |
| 1) After power cycle and comreset, SET MAX ADDRESS should be completed (Toshiba issue 18) 2) reset gLastTrigWrite = 0 in SataErrorHandleforHostW() (Netcom issue, nVidia MCP78V) |  |  |
| K1222A |  | ISP2250new.BIN |
| 1.wait all flash busy done before leaving write loop for dummy program(Toshiba issue 19,20) |  |  |
| K1227A |  | ISP2250new.BIN |
| 1. Fix side effect of only one read/write command queue(bug in K1220A) 2. Fix hang-up problem in power mode(bug in K1212A) |  |  |

| SW 變更事項 | Controller | 檔名 |
| --- | --- | --- |
| K1123A |  | sm22XMPToolK1123.exe |
| 1. Modify "DoDownloadISP" procedure, to add error handle for free memory and modify write vendor command. |  |  |
| K1128A |  | sm22XMPToolK1128.exe |
| 1. modify 0x00 to 0x20 for firmware version space. |  |  |
| K1201B |  | sm22XMPToolK1128.exe |
| 1) add CID option [0x11] CPU clock setting |  |  |
| K1204A |  | sm22XMPToolK1204.exe |
| 1. Fix Bug of QC Check CID fail. |  |  |
| K1208A |  | sm22XMPToolK1208A.exe |
| 1. Modify check block rule when setting capacity manually. |  |  |
| K1209A |  | sm22XMPToolK1209A.exe |
| 1) Add save RDT file button for Toshiba request, for multi port saving, filename is "SN".txt, save into folder "RDT File" |  |  |
| K1216A |  | sm22XMPToolK1212A.exe |
| 1) Modify name of related RDT info . |  |  |
| K1220A |  | sm22XMPToolK1220A.exe |
| 1. add CID option [0x00] bit3 , Delay SATA signature check box 2. add “Early retire” setting. |  |  |
| K1221A |  | sm22XMPToolK1221A.exe |
| 1. add CID option [0x38] , RDT Hours. |  |  |
| K1222A |  | sm22XMPToolK1222A.exe |
| 1. add Check Early Retire and Move Enable/Disable |  |  |