慧榮科技股份有限公司

研發設計/變更版本管制表單				單版次:A

| 產品名稱 : SM2258 (Project leader : Ken Kuo) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 用途: |  |  |  |  |  |  |  |  |  |
| 隸屬專案: SM2258 |  |  |  |  |  |  |  |  |  |
| 此版本為: □新發行 ■設計變更 |  |  |  |  | 日期: 10/20/2016 |  |  |  |  |
| 資料類別歸屬: □IC H/W ■IC F/W □系統H/W □系統 F/W ■應用程式 □其他 |  |  |  |  |  |  |  |  |  |
| 版次: | #### H/W版次 |  | F/W版次 |  |  |  | S/W版次 |  | Check Sum |
|  |  |  | ISP2258 P0422B |  |  |  | P1019A |  | 0x021287F4 |
| 內容概述: F/W: S/W: 1. Fixed and modified Debug mode GetArg function. 2. Add two CID option LPDDR and DRAM ECC option. Add showing option in setting.set 3. In debug mode , add button Read falsh 0B CMD. 4. In Reading card mode, modified internal interleave way display 5. Add Option, specialoption for Transcend ATA Security. |  |  |  |  |  |  |  |  |  |
| 建議通知客戶群組 |  |  |  |  |  |  |  |  |  |
| 需知會相關人員 |  | David, Derek,Brian,Steve,Eric,Aboson |  |  |  |  |  |  |  |
| 部門主管 |  | 單位主管 |  | 設計/變更 |  | 功能驗證 |  | 經辦人 |  |

接前頁內容概述：

| Program 變更事項 | Controller | 檔名 |
| --- | --- | --- |
| P0201 |  |  |
| MPTool New Firmware ISP |  | sm22XMPToolP0421A.exe ISP2258.BIN |

| FW 變更事項 | Controller | 檔名 |
| --- | --- | --- |
| P0421A |  | ISP2258.BIN |

| SW 變更事項 | Controller | 檔名 |
| --- | --- | --- |
| P0421A |  | sm22XMPToolP0421A.exe |
| 1. For Down Grade NAND Flash, add function to combine sorting bad block information. 2. Add for new Database field, 2D NAND flash or 3D NAND Flash 3. New TLC type 13:Hynix 3D TLC, 14:Toshiba 3D TLC , it only get TLC Seed table for 3D TLC, it doesn't need ProgOrder2WordLine either ProgOrder2WLOrder. 4. For SM2256 download ISP procedure, add internal chip count exceed 2 case. 5. Add new CID option "Mark Bad when do RAID" , it is only for FW Debug using, not for customer and add option to hide or show. 6. Add the judgment of ISP mode when ForceROMcode function 7. For SM2258 3D NF, on Pretest header, modified the TLC Seed table offset and data lengthes. 8. When enable SM2258_3DNF_ISP option, it won't erase ISP block. 9. Add new case of page per block on pretest header. 10. New definition of block number per chip on pretest header 0x37/0x38 11. Hide Early retire/Move TH. and Read retry iteration CID option for SM2256. |  |  |