Nvme-SM2260 User manual

of parsing Event Log Info

Updated date: 2018 / 06 / 24

Version: 1.0

# Introduce:

This doc is for SMI SM2260 get EventLot flow.

# Vendor Command Set

- Read Flash ID – C2h/40h

[Description]

This command is used to get the Flash ID of every die, current running firmware mode and parameter information, including status report of previous firmware action. The data transfer length is indicated in DWord 10 which is double word base.

[Protocol]

D2H (Device to Host)

[Input]

| OP Code | Dword 12 |  |  |  | Dword13 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 (LSB) SubCode | 1 Command Class | 2 Command Code | 3 | 4 Par Opt | 5 | 6 | 7 |
| C2h | 40h |  |  |  | 01h |  |  |  |
|  | Dword 14 |  |  |  | Dword 15 |  |  |  |
|  | 8 | 9 | A | B | C | D | E | F |

#Par Option:

This field indicates if the return data includes parameters page or not.

Please set 1 to include parameter page.

[Output Data Structure]

| Byte Offset | Description |
| --- | --- |
| 0x0003 | SectorPerPage (Multi-plane) |
| 0x01F0~0x01F7 | “SM2260AA” |
| 0x029E~0x029F | Number of page (Event Log) |

* Read offset 0x003 to get page size. (Unit: 512 bytes)

* Read offset 0x1F0~0x1F7 to check IC version.

* Read offset0x29E~0x29F(Free pointer of Log Block) to get the number of recorder.

- Read Debug Log – C2h/57h

[Description]

This command is used to extract firmware internal log information.

The DWord10 field set to number of dwords in a page( (SectorPerPage * 512) / 4 ).

[Protocol]

D2H (Device to Host)

[Input]

| OP Code | Dword 12 |  |  |  | Dword13 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 (LSB) SubCode | 1 Command Class | 2 Command Code | 3 | 4 | 5 | 6 | 7 |
|  |  |  |  |  | Page Index |  |  |  |
| C2h | 57h |  |  |  | 00000000h ~ Free pointer of Log Block |  |  |  |
|  | Dword 14 |  |  |  | Dword 15 |  |  |  |
|  | 8 | 9 | A | B | C | D | E | F |

#Page Index:

This field indicates the page number of log block

[Output Data Structure]

Please refer to Event Log Format

- Event Log Map

- Parsing rule

For this part, need to refer Logsetting.dat to translate information.

[For Example]

<A>Message ID = 0xA0, 0x01, with 1 parameter (4 bytes)

<B>Message ID = 0x83, 0x29, with 2 parameter (4 bytes / 2 bytes)

[Format Specifiers]

<x> or <X> is a format specifier that format and output the hexadecimal value. (Uppercase)

- %02X means if output value is less than 2 digits then 0 will be prepended.

- %04X means if output value is less than 4 digits then 0 will be prepended.

- %06X means if output value is less than 6 digits then 0 will be prepended.

- %08X means if output value is less than 8 digits then 0 will be prepended.

- %0AX means if output value is less than 10 digits then 0 will be prepended.

- %0CX means if output value is less than 12 digits then 0 will be prepended.

- %0EX means if output value is less than 14 digits then 0 will be prepended.

<d> or <D> is a format specifier that format and output the decimal value.

- %02d means if output value is less than 2 digits then 0 will be prepended.

- %04d means if output value is less than 4 digits then 0 will be prepended.

- %06d means if output value is less than 6 digits then 0 will be prepended.

- %08d means if output value is less than 8 digits then 0 will be prepended.

- %0Ad means if output value is less than 10 digits then 0 will be prepended.

- %0Cd means if output value is less than 12 digits then 0 will be prepended.

- %0Ed means if output value is less than 14 digits then 0 will be prepended.