JMicron 60X MP tool (Ver. B.2.2)

Draft, 2008.12.08

1. Feature:

- Integrate download firmware & read/write testing.

- Concurrent multi-port process.

- Jumperless setting.

- Provide windows user interface and easy to use.

- Auto-selection of system file.

- Auto detect the configuration of flash array and update to INI file.

- Use one button to complete the preformat, download system firmware and read/write test.

- Interface : Data access through USB or USB to SATA bridge(JM20329) or USB to PATA bridge(JM20335).

- Input device information such as serial number, model name…etc.

- Read/write defect table.

- Error proof mechanism.

2. Functions:

2-1. Test item: can select partial test items to do.

2-2. Flash info: disk media setting include flash maker, type, bank number…etc

2-3. Device setting: Input serial number, model name, f/w version, VID/PID…etc.

2-4. F/W setting: Show firmware setting.

2-5. RD: can get special information to analyze.

2-6. H/W setting:

2-1. Test item

- Sata interface test.

- Flash interface test.

- Build defect table.

- Pre-format.

- Download program.

- Read/write test.

- Format(FAT32)

2-2. Flash setting

- Flash maker.

- Flash type.

- Bank number.

- Channel number.

- Select specified firmware.

- Set drive current

2-3. Device setting

- Input serial number, model name, f/w version, VID/PID…etc.

2-4. FW setting

- Later to early: add early bad blocks to early bad blocks when create defect table.

- Check flash module: check flash ID, max. channel number and max. bank number.

- Erase rebuild early bad : Use the erase method to rebuild early bad blocks.

- Repair used module : Preformat the module with new flash and used flash.

- Recover code block : Erase the code block first.

2-5. RD

- Get defect table from flash.

- Put defect table to flash.

- Rebuild bad block mark

2-6. H/W setup

- Via JMF329 USB to SATA bridge board (PCB No. JM329-DB-001-0) to do all test.

USB to SATA bridge board ↑

Bridge board connect to device ↑

3. Notice:

- On EV board, you can always set GPIO5=0 (flash download) to download and test. But when the downloaded firmware is invalid and can’t accept ATA vendor command 0xC1(Jump to loader), you may cause system fail. If you want to download f/w again, you have to set GPIO5=1 (host download) to download a valid firmware.

- Flash test may destroy system blocks existed in flash. Don’t use MPTool to test flash only.

- When MPVersion = 0 in INI file, user can modify test setting

When MPVersion = 1 in INI file, only <Start> <Refresh> <Close> button can be pressed. If you want to unlock the limit, first set on <RDVersion> checkbox and set password “jmicron”, then you can select any button.

- If any fail has caused, you can double click the fail line to get error message.

- “Connect check” combo box provide user to check if the number of detected devices do not match the input value.

- “Red circle button” is used to automatically detect and update flash settings on factory mode. The password is “jmicron”. If this process is successful, the button will turn to green.

- You can set USB device to removable media device from [UsbRemovable]=1 of INI file.

4. Error code:

| Test item | Error code |
| --- | --- |
| Check flash module fail | M00 |
| Sata interface test | A00 |
| Flash interface test | B00 |
| Pre-format | C00 |
| Download program | D00 |
| Read/write test | E00 |
| Format | F00 |

5. INI file definition:

|  | Type | Description |
| --- | --- | --- |
| [Setting] |  |  |
| ProductModel | String | Show the product model. Default value “JM” will hide. |
| MPVersion | 0/1 | ‘1’ for Factory MP, 0 for RD setting. |
| ShowTitle | 0/1 |  |
| [TestItems] |  |  |
| EnFlashModule | 0/1 | Enable flash module test |
| EnSataTest | 0/1 | Enable Sata Test |
| EnFlashTest=1 | 0/1 | Enable flash Test |
| EnPreFormat=1 | 0/1 | Enable pre-format process |
| EnDownload=1 | 0/1 | Enable download code process |
| DownloadType=1 | 0/1 | Download SATA /USB code (SATA: bit0,USB:bit1) |
| EnRWTest=0 | 0/1 | Enable read/write test |
| EnFormat=0 | 0/1 | Enable format process |
| DiskFormat=0 | 0/1 | Format file system type select (NTFS: 0, FAT32: 1) |
| DiskLabel | String | Disk volume label assign (FAT32 at most 11 characters) |
| SaveDefTable=0 | 0/1 | Save Def. Table |
| [DeviceSetting] |  |  |
| ModelNum | String | SATA identify model number |
| FirmwareVer | String | SATA identify firmware version |
| SerialNum | String | SATA identify/USB serial number |
| SNAutoInc | 0/1 | Serial number auto increase |
| SNDecimal | 0/1 | Serial number use hexadecimal(0)/decimal(1) |
| SNErrReuse | 0/1 | Reuse serial number of error device. (0: assign new SN for each device. 1: use last fail device’s SN if had.) |
| VID=152D | String | USB VID |
| PID=0602 | String | USB PID |
| ManufName | String | USB manufacture string |
| ProductName | String | USB product string |
| [FlashSetting] |  |  |
| Capacity | Dec. | Flash capacity check (2 n Gbytes) |
| FlashName | String | Stored flash name selection |
| Channel | Dec. | Stored channel selection |
| Bank | Dec. | Stored bank selection |
| DrvCurrent=0 | Dec. | Store drive current selection |
| ExtFlashFile=flash.ini | String | Flash attributes setting file (Must have) |
| [FwSetting] |  |  |
| DefPercentage | Dec. | Defect block ratio |
| ResPercentage | Dec. | Reserve block ratio |
| Bch8ReadErrorThreshold | Dec. | Mark block as defect when ECC fail bits over/equal the threshold in BCH 8 |
| Bch15ReadErrorThreshold | Dec. | Mark block as defect when ECC fail bits over/equal the threshold in BCH 15 |
| Bch8EccErrorCopyThreshold | Dec. | Copy data to good block when ECC fail bits over/equal the threshold in BCH 8 |
| Bch15EccErrorCopyThreshold | Dec. | Copy data to good block when ECC fail bits over/equal the threshold in BCH 15 |
| WearLevelFrequency | Dec. | Difference of average and max erase count is the value x2 |
| SmartEnable | 0/1 | Enable SMART feature |
| SecurityEnable | 0/1 | Enable security feature |
| SlumberEnable | 0/1 | Enable slumber feature |
| InterleaveEnable | 0/1 | Enable interleave feature |
| ScrambleEnable | 0/1 | Enable scramble feature (Internal mechanism) |
| RemapEnable | 0/1 | Enable remap feature (Internal mechanism) |
| ParallelMemEnable | 0/1 | Enable parallel memory feature (Internal mechanism) |
| SecurityEraseEnable | 0/1 | Erase all addressable data when GPIO 1 = 0 |
| UsbRemovable | 0/1 | 1=Set USB device to removable media device. |
| UsbLedP14 | 0/1 | 1=set GPIO#14 as USB LED, 0=set HDDA as USB LED. |
| [RDSetting] |  |  |
| TableIn | String | Defect table save to |
| TableOut | String | Defect table write from |

6. Illustrattion:

- Test items

Double click this line to get original firmware version.

Download file name (SATA file/USB file) & version

You can select partial test items to execute.

First to identify tested device.

Current secteted configuration file name

- Flash setting

Auto detect flash array configuation.

Select correct flash setting

Select correct capacity depended on flash setting.

If you don’t want to check it, select “don’t care” item.

IO drive current setting

- Device setting

USB device info

Automatic increase serial number

Model name for SATA device & inquiry data for USB device

- F/W setting

See section 2-5.

These setting don’t save to INI file when close application.

Refer to .INI definition.

- RD setting

Read/write defect table

Rebuild flash map with bad block mark for defect table creation.

  - Get error message

Double click this line to get the detailed message

7. History:

| Version | Date | Firmware version | Remark |
| --- | --- | --- | --- |
| B.1.0 | 08/07/22 | 080717 |  |
| B.1.5 | 08/08/29 | 080722 | 1.check device count 2.error message process. |
| B.1.6 | 08/09/01 | 080826 | 1.disable R/W test & format item on USB download mode. |
| B.1.7 | 08/010/03 | 080827 | 1.support 4bank with Bank#0/#4/#8/#12 layout configuration. 2. use “PIO mode” to download BIN file into controller. |
| B.1.8 | 08/010/07 | 080827 | 1.auto update flash info to mptool.ini 2.support new flash : G4S, TAA1, UAG, UBG, UCG 3.new f/w parameter structure (copyback, security erase) 4.backward compatible(<080924,>=080826) |
| B.1.9 | 08/010/16 | 081016 | 1. support new flash : G6D, FMC |
| B.2.0 | 08/011/03 | 081028 | 1. download USB binary code only through USB interface. 2. set USB device to removable media device. |
| B.2.2 | 08/12/08 | 081208 | 1.support the function of format(FAT32). 2.support the feature of usb led. |